;-----------------------------------------------------------;

; DSKL.C          DSK Loader program                        ;

; (C) 1992-93                                               ;

; Texas Instruments Inc.                                    ;

; By: Keith Larson                                          ;

;     TMS320Cxx DSP Applications                            ;

;-----------------------------------------------------------;

DSKL is written to show how low level RS232 interfacing to the TMS320C26

is performed.  It contains the neccessary code to read DSK files and bootload

them into the device as well as communicate with a communications kernal

DSK_COMM.  From this basic capability it is possible to load other programs

which run interactively with DSKL and DSK_COMM.

Two programs which have been written specificly to work with DSKL are

AUTOTEST.ASM and HOSTSPEC.ASM and are described below.

RUNNING AUTOTEST

================

1.  Your computer can be left on when changing DSK's

2.  The 9VAC power transformer can be left plugged in the wall

3.  Connect the RS232 link from the host PC/AT to the DSK under test.

4.  Plug the 9 VAC power conector into the DSK.  The LM7805 voltage

regualtor will get warm if power is correctly applied.

5.  Go to the DSK test directory using the appropriate DOS commands

6.  Start the test by typing:

C:\DSKTOOLS\DSKL -A -C1          (If using comport 1)

C:\DSKTOOLS\DSKL -A -C2          (If using comport 2)

The parameters passed to DSKL tell the program to automaticly

boot the DSKL communications kernal (DSK_COMM.DSK), display some

help screens (Q will exit help), and then load the file AUTOTEST.DSK.

DSKL then enters a host echo mode acting as a user shell to the

AUTOTEST program.  When properly loaded, AUTOTEST.DSK will send

diagnostic messages to the host which are echoed to the screen.

AUTOTEST executes in a continuous loop.  To exit hit [Q].

7.  After each message you will need to press >enter< to proceed

8.  A good DSK will respond with the following messages

DSK-RS232 Loop                   OK     >enter<   ;

C26 MEM Check: 0x0060  0x0080    OK     >enter<   ; Checking RAM B2

C26 MEM Check: 0x0400  0x0800    OK     >enter<   ; Checking RAM B1-B3

AIC Loopback :                                    ;

;

0x7000  0x66A0                                    ; Checking 3 point

0x9000  0x95DC                                    ; linearity

0x0000  0xFE4C                                    ;

0xFFF2                                            ; Linearity Error

;

AIC Loopback : OK     >enter<                     ; AIC Pass

9.  If any of the tests fail a  'BAD >enter<' message will be generated

along with an audible beep from the host PC/AT.  If the RS232 link

cannot be established, DSK_COMM will likely not load and AUTOTEST

will not be attempted.  If there are problems, go to the section

"DSKL COMMUNICATIONS AND SUB-FEATURES" in this document.

DSKL COMMUNICATIONS AND SUB-FEATURES

====================================

1.  If the baud rate is too high, or the wrong commport is selected, the

DSKL will eventually cycle to a different comport and baud rate

In some cases, such as high noise, long cables etc..., the TMS320C26

RS232 Autobaud routine may not be completely synchronized. In this

case a lower baud rate may be desirable, even though the DSK will

initialize at a higher rate.  Boot failure or erratic performance

can be caused by...

a) Wrong port selected (after 8 tries, the comports are cycled)

b) RS232 cable (or connection) bad or noisy

c) No power (9 VAC transformer UL CLASS 2 only!).  If power is getting

to the DSK the LM7805 voltage regulator will be warm to the touch

d) Baud rate too high (after 16 tries, the baud rate will be lowered)

e) DSK_COMM.DSK or AUTOTEST.DSK not in directory

f) AUTOTEST.DSK is loaded using DSK_COMM.DSK which must be bootloaded

before entering AUTOTEST.  Bootload initialization of DSK_COMM

is automatic the first time, but may be corrupted by user operations.

g) The DSK is broken

2.  DSKL can also examine the DSK's internal memory as well as load and

execute files.  If higher levels of debug support are needed, the file

DSKD.EXE can be executed.  Supported DSKL commands (while NOT in

AUTOTEST mode) include

A  - Autotest DSK          Loads and executes AUTOTEST.DSK

Q  - Quit DSKL

R  - Reset DSK (no load)

I  - Boot (initialize) DSK_COMM.DSK

B  - Bootload file         <file>

L  - Load file                               << Using DSK_COMM

X  - Load and execute file <file>            << Using DSK_COMM

E  - Execute @             <address>

Dx - Dump Data/Prog        <address> <length>

Fx - Fill Data/Prog        <address> <length> <value>

G  - Graph data received   (EGA, signed char, 0=synch)

H  - HELP menu (this)

3. DOS COMMAND LINE USAGE FOR DSKL

C:\DSKTOOLS\DSKL [I] [A] [Cx] [Bxxxxx]

|   |   |    +-Baud xxxxx (9600,19200,38400)

|   |   +------Initialy load to COM1, COM2

|   +----------Autotest

+--------------Invert the DTR signal

NOTE:

After  8 initialize failures, the commport will toggle

After 16 initialize failures, the baud rate will be halved

===============================================================================

===============================================================================

HOSTSPEC.ASM: A spectrum analyzer using your HOST graphics display!

===================================================================

NOTE: HOSTSPEC.ASM will NOT work with DSKD!  It is designed to work with

the DSK loaders (DSKL) [G]raphics option.

QUICKRUN:

To run this program, first assemble it using DSKA. Then start DSKL,

select 'X' for load and execute.  Then select 'G' for graphics.  To exit,

hit any key (except Q).  Since the keyboard is not flushed, 'Q' will

quit DSKL entirely!

For further information read HOSTSPEC.DOC