+--------------------------------------------------------------------+

|              A Spectrum Analyzer for the TMS320C26                 |

+--------------------------------------------------------------------+

Written By: Keith Larson

TMS320Cxx DSP Applications Engineer

Texas Instruments Inc

OVERVIEW

--------

This application is an extremely compact and efficient real-time FFT

spectrum analyzer for the TMS320C2x and TMS320C5x family of processors.  The

analyzer is specificly written to operate on the TMS320C26 DSP Starters Kit

(TMS320C26 DSK), but can easily be modified to run on other platforms and

devices.  Analog input and output is through the DSK's TLC32040 AIC (Analog

Interface Circuit) which has both A/D and D/A capabilities.  The spectrum

is then viewed with an oscilliscope at the D/A output, which is a synchronous

representation of the input signals spectrum.

RUNNING THE ANALYZER FOR THE FIRST TIME

---------------------------------------

To use the analyzer with the DSK, simply assemble 'DSK_SPEC' using the

DSK Assembler and then load it into the DSK using the DSK debugger.  If your

DSK is connected to your RS232 commport, is powered up, and you have an

oscilliscope connected to the output, you are ready to go.  Here are the

steps to run the analyzer...

From the C:\DSKTOOLS directory (or have it in your path) enter....

C:\DSKTOOLS\DSKA DSK_SPEC      <== This assembles the file DSK_SPEC.ASM

C:\DSKTOOLS\DSKD [c2]          <== Bring up the debugger (commport 2 if needed)

LD DSK_SPEC                    <== Load the analyzer

F5 (Function Key 5)            <== Run the analyzer

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

>>      Wait a few seconds for the DSKD to initialize.        >>

>>      If it does not, check the following...                >>

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

1) Check the RS232 cable and connectors

2) Is your 9VAC transformer plugged in at both ends? (UL-CLASS 2 ONLY!)

If power is getting to the DSK, the 7805 voltage regulator will be warm.

3) Try a lower baud rate.  Some portables, Windows and OS/2 have low level

software limitations that often limit the baud rate.

4) Is the oscilliscope in NORM/AUTO mode?  You may need to experiment with

the triggering and levels to get a good display.

BLOCK DIAGRAMS

--------------

To better understand the interlocking nature of the analyzers routines,

block diagrams will be used.  There are several 'tricks' which have been

imbedded into these routines that are described in detail following the

block diagrams.

MAIN PROGRAM FLOW

-----------------

Step 1)  The DSP's synchronous serial port and the AIC are initialized.

Step 2) Initialize the FFT runtime parameters.  This includes the size of

the FFT and where the FFT data arrays are located.  During each

loop, this data is not returned in the same state as it begins.

Step 3) Compute the FFT of the data array.  (See FFT flow diagram)

Step 4) Calculate the log magnitude of each element of the FFT array.  This

effectively reduces the REAL/IMAG pair to a single value, magnitude,

which is stored in the REAL position.  The actual log base is 2, but

is not relavant to the output view (oscilliscope).  This is because

other log bases (including dB) are nothing more than a scaler

multiplication.  That is, you have to adjustment the volts/division

of your oscilliscope if you want a calibrated output!

Step 5) Perform an in place bit-reversal of the REAL data (log magnitude)

into the now redundant IMAG data.

Step 6)  FFT/IO BUFFER INTERLOCK!

When the I/O buffer finally fills, execution is allowed to continue.

At this time a large negative D/A value (video synch) is sent to

the D/A signaling the beginning of a new spectrum.

NOTE, the RINT ISR is turned off when the buffer is filled.  This

allows the swap routine to complete at least one swap (first data

point) before the interrupt is re-enabled.

The complete I/O buffer data is then swapped with the FFT's

converted data (new spectrum).  As the I/O array is being copied

to the FFT array it is also multiplied by a raised cosine window

derived from the FFT twiddle lookup table.

Step 7)  Go back to step 2.

+---------------+

|     START     |

+---------------+

V

+---------------+

|   INITIALIZE  |

| SAMPLE FREQ & |

| INTERUPT MASK |

+---------------+

+---------------->>|

|                  V

|          +---------------+

|          |   INITIALIZE  |

|          | FFT PARAMETERS|

|          +---------------+

|                  V

|          +---------------+

|          |  COMPUTE FFT  |

|          +---------------+

|                  V

|          +---------------+  +-----------+

|          | CALCULATE     |<<| LOOP TILL |

|          | LOG(RL^2+IM^2)|>>| DONE      |

|          +---------------+  +-----------+

|                  V

|          +---------------+  +-----------+

|          | IN PLACE      |<<| LOOP TILL |

|          | BIT REVERSAL  |>>| DONE      |

|          +---------------+  +-----------+

|                  V

|          +---------------+  +-----------+

|          |   I/O BUFFER  |<<| WAIT TILL | SEE FFT INTERLOCK

|          |     FULL?     |>>| ISR DONE  |

|          +---------------+  +-----------+

|          |  SYNCH PULSE  |

|          +---------------+

|          |    SWAP ONE   |

|          +---------------+

|          |  ENABLE RINT  |

|          +---------------+  +-----------+

|          | WINDOW INPUT  |<<| LOOP TILL |

|          | AND SWAP WITH |>>| DONE      |

|          | FREQUENCY DATA|  +-----------+

|          +---------------+

|                  |

+------------------+

RINT INTERRUPT SERVICE ROUTINE FLOW DIAGRAM

-------------------------------------------

The following diagram shows how each new data point is processed by

the RINT interrupt service routine.  When an interupt occurs the ISR writes

the frequency information in the current buffer position out to the DAC.

Step 1)  On entry, the global interrupt enable is turned off by the CPU.

The status registers are saved, some initialization is performed,

and a check is made of the current I/O buffer position.

Step 2)  Is the buffer filled?

Step 3a) If the buffer is full, the receive interrupt is turned off reducing

overhead for faster FFT calculation. When the FFT is done, RINT is

re-enabled (by the FFT routine) to collect more data.  This will only

happen if the incoming data is received at a rate faster than the FFT

can process it.  This is about 50 KHz for the DSK.

Step 3b) If the buffer is not full, then the ISR outputs a spectrum value from

the circular I/O buffer, and gets a new value from the A/D to take

it's place.  The I/O buffer pointer is incremented and RINT is turned

back on.

Step 4)  Execution returns to the main routine (FFT) after restoring the

original CPU context.

+---------------+

| INT RECEIVED  |

| INT MODE=OFF  |

+---------------+

V

+---------------+

| SAVE CONTEXT  |

+---------------+

V

+---------------+

+----------+IS BUFFER FULL?|

|     YES  +---------------+

|                  V NO

|          +---------------+

|          |    OUTPUT     |

|          | FREQUENCY DATA|

|          +---------------+

|                  V

|          +---------------+

|          | INPUT NEW DATA|

|          +---------------+

|                  V

|          +---------------+

|          | INCREMENT     |

|          | BUFFER POINTER|

|          +---------------+

|                  V

|          +---------------+

|          |    ENABLE     |

|          |   INTERUPTS   |

|          +---------------+

|                  V

+------------------+

(RINT OFF)     +---------------+

|RESTORE CONTEXT|

+---------------+

|

+---------------+

|    RETURN     |

+---------------+

FAST FOURIER TRANSFORM EXECUTION FLOW DIAGRAM

---------------------------------------------

The FFT used in this application is a complex radix-2 decimation in

frequency FFT.	Decimation in frequency was chosen in this case because

of the decreasing block size with each succesive stage of the FFT by one

half.  Since a change of 1/2 is a shift right it would make sense that

this could be an advantage.   This can be easily done using the TMS320C26's

indirect addressing modes.  This is explained in detail in the TMS320C2x

Users Guide and in one of the following examples showing a special BIT

Reverse Addressing trick.

With this simple concept in mind, and a quick check of the FFT's flow

diagram you will quickly notice that if you load AR0 with FFT_size/2

and repeatedly execute a *BR0+ for every stage, AR0 can be very handy.

In particulr AR0 can be used to address FFT data, count the number of

DFT blocks, address twiddles.  Furthermore, AR0 will eventually go to

zero making a simple BANZ test all that is needed to break the sequence!

THE BIG ADVANTAGE >> If you want to change the size of the FFT, simply

change the starting value of the FFT size.  If the new size is smaller

than the maximum allowed by the size of the twiddle array, then the

same tables can be used again.  This makes this routine particularily

usefull in high level language programming where it is difficult to

reload arrays each time an FFT is needed!

stage 1      stage 2      stage 3           end

------       ------       ------

|      |     |      +-----+      +---

|   8  +-----+      +-----+  x2  +---

| POINT+-----+      |      ------

---+  DFT +-----+  x4  |      ------

---+      +-----+      +-----+      +---

---+      |     |      +-----+  x2  +---

---+  x8  |      ------       ------

---+      |     |      +-----+      +---

---+      +-----+      +-----+  x2  +---

---+      +-----+      |      ------

---+      +-----+  x4  |      ------

|      +-----+      +-----+      +---

|      |     |      +-----+  x2  +---

------       ------       ------

AR0=4        AR0=2        AR0=1      AR0=0

Block       8            4            2          1

Step 1) Load the size of the FFT to run into AR0 (max on DSK is 256!)

(This is the only step that a user really needs to know!)

Step 2) Load AR2, AR4 and AR5 with the base addresses of the I/O data array,

Twiddle array and 1/2 FFT size, respectively

+---------------+

|   FFT START   |

+---------------+

V

+---------------+

|   AR0=FFT_S/2 |

+---------------+

|<<-------------------------+

V                           |

+---------------+                   |

| AR2=AR3=D_base|                   |

| AR4=T_base    |                   |

| AR5=FFT_S/2   |                   |

+---------------+                   |

|<<------------------------------------+

V                           |          |

+---------------+                   |          |

| TOP=AR2+2*AR0 |                   |          |

+---------------+                   |          |

V                           |          |

No (more DFT's)+---------------+                   |          |

+------------+    AR5 = 0 ?? |                   |          |

|            |DFT block count|                   |          |

|            +---------------+                   |          |

|                    |                           |          |

|                    V Yes (new stage)           |          |

|            +---------------+                   |          |

|            |  AR0 >> 1     |                   |          |

|            +---------------+                   |          |

|                    V                           |          |

|            +---------------+ No                |          |

|            |    AR0 = 0 ?? +-------------------+          |

|            |    FFT DONE   |                              |

|            +---------------+                              |

|                    V Yes                                  |

|            +---------------+                              |

|            |    FFT DONE   |                              |

|            +---------------+                              |

|                                                           |

+---------------------                                      |

|                                      |

v                                      |

+---------------+                              |

|Fix 1st TW addr|                              |

+---------------+                              |

|                                      |

v                                      |

+---------------+                              |

|     AR6=1     |                              |

|Set DFT counter|                              |

+---------------+                              |

|                                      |

v                                      |

+---------------+                              |

|AR6=AR6*BR0+AR0|                              |

+---------------+                              |

|<<---------------------+              |

|                       |              |

v              +----------------+      |

+---------------+      |New twiddle and |      |

|Calculate BFLY |      |Top and Bottom  |      |

|Top & Bottomm  |      |data addresses  |      |

|add and subt   |      +----------------+      |

+---------------+               |              |

|              +----------------+      |

|              |Multiply twiddle|      |

v              +----------------+      |

+---------------+ No            |              |

|    AR6=0 ??   +--->>----------+              |

| (last TW addr)|                              |

+---------------+                              |

Yes |                                      |

v                                      |

+---------------+                              |

| Twiddle addr  |                              |

| reinitialized |                              |

| Modify BOT ptr|                              |

+---------------+                              |

|                                      |

+--------------------------------------+

SPECIAL TIPS AND TRICKS USED IN DSK_SPEC.ASM

============================================

USING THE INDIRECT BIT-REVERSED MODE FOR LOGICAL SHIFTS

-------------------------------------------------------

A feature which has been extensively used to reduce code size is bit-reversed

addition in the auxiliary registers.  This eliminated much of the usual loop

overhead needed for the FFT as well as make accessing the twiddle and window

coefficients easy.  Because of this the tables cannot be located anywhere in

memory.  The twiddle tables MUST be located on an boundary twice the size

of the FFT to be performed.  This is explained further in the TMS320C2x users

guide.

This special case of indirect addressing comes down to the following two

examples.  The first is that if you add something to itself it's value will

double. This is a left shift.  Since bit reversed addition is actually a

mirrored image of normal addition, the result of adding something to itself

will be a right shift as shown below.

Normal AR0+AR0 addition

lrlk  AR0,080h   ;AR0=0000000010000000

larp  AR0        ;ARP=0

mar   *0+        ;AR0=0000000100000000   <Note:left shift by 1

mar   *0+        ;AR0=0000001000000000

mar   *0+        ;AR0=0000010000000000

mar   *0+        ;AR0=0000100000000000

mar   *0+        ;AR0=0001000000000000

mar   *0+        ;AR0=0010000000000000

etc...

Bit reversed AR0+AR0 addition

lrlk  AR0,080h   ;AR0=0000000010000000

larp  AR0        ;ARP=0

mar   *0+        ;AR0=0000000001000000   <Note:right shift by 1

mar   *0+        ;AR0=0000000000100000

mar   *0+        ;AR0=0000000000010000

mar   *0+        ;AR0=0000000000001000

mar   *0+        ;AR0=0000000000000100

mar   *0+        ;AR0=0000000000000010

etc...

(NOTE: This is also helpful as a sliding bit test in succesive approximation!)

VERY FAST LOGARITHMS FOR DB SCALED OUTPUT

-----------------------------------------

Another feature not commonly found in FFT code is dB (logarithmic) scaled

output.  In DSK_SPEC a logarithm (scaled to fit 16 bits) is performed by simply

normalizing the result of (IMAG*IMAG + REAL*REAL) to a floating point value.

The normalized mantissa is then simply concatenated to the shift count before

storing the result back into the FFT data array.  The result is a logrithmic

number (base 2) accurate to within +6.5/-0% of the desired result.

EFFICIENT CIRCULAR I/O BUFFER

-----------------------------

By using the RINT ISR for both input (analog) and output (spectrum) data, both

are updated at the same rate and need 1/2 the space.  However this means that

the input and output rates are locked together.  For very slow data rates, the

oscilliscope sweep rate will be low and the synch pulse relatively narrow

compared to the spectrum.  You may want a digital, and or storage oscilliscope.

At very high rates, the FFT execution time will dominate.  In this case, the

synch pulse will be relatively wide and there may be a 'pedistal' effect as the

FFT catches up to the data (this will not be a problem with the TLC32040 AIC!).

CONCLUSION

----------

By incorporating all of the tricks shown above, it was possible to create a

full featured spectrum analyzer that is very compact and efficient.  No special

hardware or additional software is needed, other than a host PC and oscilliscope

to run the program.  Additionaly, the methods and routines described can be

easily incorporated into many other programs with little effort.