FFTDSP Documentation

Version 3 - March 2, 1993

PROGRAM DESCRIPTION

FFTDSP is a Digital Signal Processing (DSP) program which can detect

very weak signals from a Radio Receiver using realtime Fast Fourier Transforms

(FFT).  Amplitude levels for each FFT frequency are converted to color and

displayed as a continuous spectrograph.  The resulting screen simultaneously

shows the output of 640, 2 Hz wide filters between approximately 300 and 1500

hertz.  Each horizontal line represents the output of all 640 filters during a

0.5 second period.  Over time, as each line is added to the screen, the eye

can begin to see weak signals emerge from the darker background.  These weak

signals may not be audible during a short listening period.

Primary Features of the FFTDSP program are:

- Realtime 4096 Point FFT for 2Hz Filtering

- Signal Amplitude Conversion to color for continuous spectrograph

- Numerical Frequency display of Max Amplitude Signal (2 Hz resolution)

- Input Audio Signal Strength Meter for setting level from receiver

- Color Level adjustment for color conversion range

- Spectrograph screen Save and Display in standard GIF format

- Mouse point and click for Frequency Measurement and Offset

SYSTEM REQUIREMENTS

- 386, 20 Mhz with Coprocessor (Minimum System)

(486 recommended for full performance and future upgrades)

- Sound Card: ProSpectrum-16, ThunderCard, SoundBlaster or

SoundBlaster-Pro

- Audio line from radio to sound card record (or line) input

DISTRIBUTION & PRICE

- A fully functional, but time limited FFTDSP program is being

distributed via the public BBSs.  Without the registration file,

operation will stop after 30 seconds.

- Registration Price: $20 US, $25 Foreign (includes shipping & handling)

The registration file will allow the public FFTDSP demo program to

operate continuously.

- Available from:    Mike Cook, AF9Y

501 E. Cedar Canyon Rd.

Huntertown, IN  46748

Tel: (219-637-3399)

BBSs used by author for program updates and mail:

Midnight Escape (2400-14400 Baud)      (219) 456-4127

East Coast Atlantic BBS (2400 Baud)    (704) 284-4854

Apogee BBS (9600 Baud)                 (508) 368-4137 & 7036

******************************* NOTE **************************************

If the program halts unexpectedly with an message such as "RUNTIME ERROR at

XXXX", it is strongly recommended that the computer be forced thru RESET.

This program uses DMA and depends upon the program normal exit to turn off

DMA.  Improper program termination without computer RESET could cause file

overwrites.

***************************************************************************

MAIN SCREEN

o Level Meter:  Rectangular box located in the upper left area of the

screen to show audio input level to the Sound card.

- Green/Red bar shows signal level for both positive and negative

peaks on a logarithmic scale.

- Recommended level is mostly Green with occasional Red peaks.

- Numeric display below the box shows peak signal level (0-255)

o Max Freq: Upper Right area which displays the highest average amplitude

Frequency during each 0.5 second sample period.

- Operates like a frequency counter with 2 Hz resolution

o Mouse Operation: Used to point to area on the screen and show the

frequency and relative offset.

- Point and Click left mouse button to show frequency

- Point and Click right mouse button to show offset from last

frequency (last click from left button)

o CPU Time: Upper Right area that displays the measured CPU % time required

to process the 0.5 second sample period.

- Less than 100% is desired.  This indicates that the computer is

keeping up with the real time data coming into the Sound Card.

Any 486 computer with coprocessor will typically be below 100%.

- Greater than 100% will result in some loss of detection for

weak signals.  Every other 0.5 second sample will be used in

this case.  Time will also be compressed so that it will take

aprox 5 mins to fill the screen instead of the normal 2.5 minutes.

o Color Level: Shows current Color Level scaling factor (0 to >5000)

- Typical setting should be approximately 150 (default) if the Input

Audio level is set for mostly green with occasional red peaks.

- Use the + or - keys to change this value up or down.  The change

in level may appear slow because of the 0.5 second updates periods.

Holding down the + or - key too long will cause beeping due to

keyboard buffer overflow.

- Recommended setting is a dark blue background for optimum weak

signal detection.  See examples of demo spectrograph GIF pictures.

- Brighter colors (ie light Green, Red, Yellow) require a larger

signal level change for transition to the next color.  The total

range for the 16 possible colors is greater than 30 db.

o [F7] Save Image

- This key will store the full screen as a GIF file that can

be viewed later using standard GIF view programs.  A program for

viewing previously stored spectrographs is available under

the [F1] utilities.

- After the image is stored and the operation continues, a

horizontal white line is drawn across the screen as a marker

showing where the screen save occurred.

o [C] Clear Screen

- This key will clear the screen and restart the spectrograph

at the top.

o [Alt-X] Exit

- Terminates the program and returns to DOS

o Frequency Scale

- Horizontal scale across the screen showing Frequency in Khz.

o Time Markers

- 1 Minute GMT Time marks are printed along the left side of the

spectrograph.

- Normally 3 Time marks will be shown since a full screen represents

a little more than 2 minutes.  On slower computers, more time marks

may be seen due to a compressed time scale. (See CPU Time)

[F1] UTILITIES

Selecting the F1 Utilities from the Main screen will change to a

secondary screen which displays additional program utilities.  While

in the secondary screen, the FFT processing will be suspended.  Returning

to the Main screen will continue the FFT processing with a horizontal white

line showing where the interruption occurred.

o [F8] View Stored Picture (GIF) File

- This key will shell to a simple GIF viewer program called

GIFVU.  Names of previously stored GIF files will be shown

and can be selected for display.  Also see Note 1.

o [F9] VU Meter (toggle)

- This key toggles the Signal Level Meter on and off.  It may

be desirable to cut off the Meter if the CPU Time is just

above 100% to improve the processing speed. (See CPU Time)

o [F4] Convert Local Time to GMT

- This function is useful if your computer is set for local time

instead of GMT.  A delta in hours is requested and will be added

to the computer local time so that the Time Marks will be in GMT.

o [F10] Save Sound for Selected Picture Area (N/A)

- A future option which will save the sound of a marked area in the

spectrograph as a standard VOC file.

o [F6] Change Color Ranges (N/A)

- A future option which will save FFT information and allow

modification of color threshold points.

o [F3] HiRes (640 x 400)

- This key will toggle the Main Screen between HiRes (640 x 400)

and MediumRes (640 x 350).  The disadvantage of HiRes is that

the screen will not scroll when reaching the bottom.

o [ESC] Return to Main Screen

- Upon return to the Main Screen, a horizontal white line will mark

the point where spectrograph operation was suspended.

NOTE 1: The GIFVU program provided can be run outside of the FFTDSP

program.  It can be used for viewing recorded spectrograph

GIF files.