PCAD2APR   Version 1.3

This converter will convert from a PCAD .APR aperture file to GraphiCode's

.APR aperture file (although the files have the same extension, they have

very different formats).  The converter can be used to convert single

files or groups of files.

Version 1.3 adds support for PCAD version 6.

usage: PCAD2APR <aperture file> [<outfile>]

Example:

To convert "ALIST1.APR" to "NEWALIST.APR" type:

PCAD2APR ALIST1 NEWALIST

This creates the 24 position aperture list NEWALIST.APR

To convert "ALIST1.APR" to "NEWALIST.APR" for laser plotting type:

PCAD2APR -L ALIST1 NEWALIST

This creates the 255 position "Laser" aperture list.

To convert several files at once type:

PCAD2APR *.APR or

PCAD2APR -L *.APR

GraphiCode aperture files will then be produced with the extension

.AP_  to distinguish them from the original PCAD .APR files.

Special apertures, those other than ROUND, DRAW, and SQUARE are

declared as custom apertures using "TRIANGLE.CAP".  For PCAD

version 6, we convert to TARGET to preserve any diameter info.  If

you are using one of these apertures, you will need to use APER to

create the desired custom aperture, or choose from the other aperture

types available in our system.

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS (206)771-6980.

Release 8-18-92 LAFM

Corrected Problem with overrunning aperture list