PRO2APR.C   Version 1.8

This converter will convert from the *.APT aperture file to the GraphiCode

*.APR aperture file.  This is compatible with the Protel Autotrax PCB Design

program.  The converter can be used to convert single files or groups of

files.   Version 1.5 adds the ability to process the PROTEL TRAXPLOT

"Gerber Aperture Match List" format, including multiply defined D codes.

usage: PRO2APR <aperture file> [<outfile>]

Example:

To convert the aperture file "ALIST1.APT" for use with GraphiCode

products, type:

PRO2APR ALIST1

This creates the GraphiCode aperture wheel file "ALIST1.APR".

To convert "ALIST1.APT" to "NEWALIST.APR" type:

PRO2APR ALIST1 NEWALIST

To convert several aperture files type:

PRO2APR *.APT

These are the formats of the Protel Aperture lists:

TYPE 1:

; APERTURE FILE FORMAT

;

; <draft code> <shape> <xsize> <ysize> <hole size> <use>

;

; EXPLANATION OF PARAMETERS

;

; <draft code>   Code assigned to this aperture e.g. D14

;                Note that this code must start with 'D'

;                followed by the code number.

;

; <shape>        Can be any of the following shapes:

;                         CIRCULAR

;                         RECTANGULAR

;                         SQUARE

;                         OCTAGONAL

;                         ROUNDRECT

;                         CROSSHAIR

;                         MOIRE

;

; <xsize>        size of aperture in the x direction in

;                mils (thous).

;

; <ysize>        size of aperture in the y direction in

;                mils (thous).

;

; <hole size>    size of hole in aperture in mils (thous).

;                zero if no hole in aperture.

;

; <use>          specifies what the aperture can be used for.

;                There are three possible settings:

;                   LINE   can only be used to draw lines

;                   FLASH  can only be used to flash pads

;                   MULTI  can be used for either

;                   blank  defaults to MULTI setting

;

D10 ROUNDED      50  50   0  FLASH

D11 SQUARE       50  50   0  FLASH

D12 RECTANGULAR  50 100   0  FLASH

TYPE 2:

D11   ROUNDED            10.000      10.000       0.000 MULTI

D12   RECTANGULAR        48.000      38.000       0.000 MULTI

D13   RECTANGULAR        45.000      40.000       0.000 MULTI

D14   RECTANGULAR        87.000      31.000       0.000 MULTI

TYPE 3 (TRAXPLOT "Gerber Aperture Match List"):

----------------------------------

PROTEL TRAXPLOT  Version 1.61

Gerber Aperture Match List

PCB File : \ASD\WERF\WERFE0ET.PCB

APT File : \QWRT\QWRT2.APT

Date     : 11/11/1993

Time     : 07:34:12

----------------------------------

ASSIGNING     : Text                    (18)

Stroke        : D13     Circular        (18 x 18)          0 (H)

ASSIGNING     : Track                   (6)

Stroke        : D10     Circular        (6 x 6)            0 (H)

ASSIGNING     : Arc                     (12)

Stroke        : D12     Circular        (12 x 12)          0 (H)

ASSIGNING     : Pad     Circular        (124 x 124)    PWR/GND

Paint         : D30     Circular        (120 x 120)        0 (H)

ASSIGNING     : Pad     Circular        (124 x 124)    PAS MSK

Paint         : D30     Circular        (120 x 120)        0 (H)

Change History:

release 1.8: better handling of partially defined shape types

release 1.7: convert an M x M rectangle to a square, M x M oblong to a round.

release 1.6: improved handling of the RELIEF shape using "=TH,4,...".

release 1.5: TRAXPLOT "Gerber Aperture Match List" support has been added.

release 1.4: Protel has added decimal points to the size descriptions.

release 1.3: Protel for windows V1.6 did not output the MULTI keyword.

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS at (206)771-6980.