SatSked version 1.937                12/16/1993

WA2N

and

NZ3F

What Is SatSked:

---------------

SatSked is a multiple satellite pass manager for amateur radio enthusiasts.

It is used to automatically track different satellites by switching various

ground station equipment via computer.  It is intended to be used with the

programs PB, PG, and their associated files.  All of these programs may be

obtained from AMSAT USA.

SatSked requires a registration number to run.  This number is available

from AMSAT NA, UK and DL.  Each of those organizations has set their own

price for SatSked.  All funds from SatSked go directly to the Phase 3D

construction fund.  You may reach AMSAT at the following addresses:

Radio Amateur Satellite Corp  (AMSAT NA)

850 Sligo Ave

Suite 600

Silver Springs MD

phone 301-589-6062

fax   301-608-3410

internet:  martha@amsat.org

AMSAT-DL Warenvertrieb

Reinhard Richter

Lohfeldweg 40

D-30459 Hannover

AX.25 DJ1KM @ DB0FAU.DEU.EU

CI$ 100114,756

Fax +49 511 411112

Ron Broadbent

G3AAJ

AMSAT UK

E12 5EQ

England

phone 44(0)81 989 6741

fax   44(0)81 989 3430

Where Does SatSked Get It's Pass Information From:

-------------------------------------------------

SatSked creates it's own schedule information.  The user's latitude,

longitude, and station elevation are used to determine the AOS and LOS times

of the selected satellites in the SATSKED.PRI file.  An up to date file

containing the keplerian elements of the desired satellites is necessary to

make these calculations.  This file may be in NASA 2 line, or AMSAT format.

Alternatively, there are two other methods to obtain schedule information.

The first method uses QuikTrak by N4HY, and QTSORT.EXE by KF5OJ.  The second

is to use ITPASS.EXE by KB5MU and QTSORT.EXE.  ITPASS is available on an

InstantTrack utilities disk from AMSAT USA.  Refer to appendix A in this

document for further details.

Where To Put The Distribution Files:

-----------------------------------

First designate a single directory where you will keep the group of files for

SatSked.  It will not be necessary to place this directory in the dos search

path.  This directory will be located through a master batch file that IS

located somewhere in the dos search path.  You should copy all of the files

from SATSKED.ZIP into to that directory, with the exception of the files

STRTPBPG.EXE and SS.BAT.  These two files will need to be located somewhere in

the dos search path.  In my examples, I'll use C:\SATSKED as the "SatSked"

directory.

How SatSked Works:

-----------------

SATSKED.EXE watches the dos clock until the proper time, then writes a file

named TRACK.BAT to disk.  TRACK.BAT then calls the appropriate batch files

for the satellite to be tracked.  We will now design a set of batch and "EXE"

files to generate a loop, which will track and communicate with multiple

satellites.

The main loop calls SATSKED.EXE, TRACK.BAT, and then calls itself to start

the cycle again.  From TRACK.BAT, batch files that start your rotor radio

tuning, and STRTPBPG.EXE are called.  STRTPBPG then will start either

PB.EXE or PG.EXE, depending on the presence of files to be uploaded.

Batch File Explanation:

----------------------

Batch files are command files which have the extension .BAT.  They are

created with any text editor.  Batch files can be used to execute DOS

commands, and perform a limited set of other operations.  A batch file can

be accessed from anywhere on disk, as long as the directory it is "lives in" is

part of the dos search path, set in AUTOEXEC.BAT.  I have a c:\batch

directory that contains several batch files with short names.  This allows me

to move to often used directories quickly, and  also to execute programs that

are not in the dos search path.

SS.BAT:

------

First we'll look at the master batch file, this is the one which starts SatSked.

If  you put SS.BAT in a directory that is included in the dos search path you

can start SatSked from anywhere by typing "SS".  The comments column is

not in any of  the files, it's just here for illustration.

******SS.BAT********************Comments***************************************

C:				Changes to the drive with SatSked

CD \SATSKED                     Changes to the SatSked Directory.

SATSKED                         Runs the SatSked main program

IF ERRORLEVEL 1 GOTO CLEANEXIT  If X is pressed from within SatSked, this line

will prevent the recursive call to SS.BAT from

being made.  This allows you to easily exit

from the "loop".

CALL C:\SATSKED\TRACK.BAT       After SatSked Terminates, track.bat is

"called" and executed.

C:\BATCH\SS.BAT                 Since a "call" was made to track.bat, execution

returns to SS.Bat, now we "execute" SS.BAT

again, and the cycle starts over.

:CLEANEXIT                      This is known as a "label" for the goto statement.

*******************************************************************************

Note that this runs over and over until you exit from SatSked with the X key.

When you run batch files from other batch files, you have the option of

returning, or passing control to the batch file in line.  To return to the

first batch file, use CALL filename.bat, or else just type the name without

the call statement to pass control.  You can keep transferring down from file

to file if you wish, the process is not limited to 2 levels.

TRACK.BAT:

---------

This is the file that SatSked generates.  Track.bat will be located in the

C:\SATSKED directory (or where you have SatSked located) when SatSked exits.

TRACK.BAT is also used to start the idle task function if it isenabled.

(see Idle Task: below)

*****TRACK.BAT**********************Comments:*********************************

CALL UO-22[C].BAT [X local_end_time unix_style_time inject_i_time maxPgCycles

abortIdleSeconds next_aosTime]

Calls the proper batch file to start tracking

the specific satellite UO-22.  If KO-23 was

the satellite to track, KO-23.BAT would

replace UO-22.BAT here.  In addition, it should

change the default directory to that which

contains the proper PB.CFG and PG.CFG.  If

SatSked is started with an ASCII parameter,

such as "SATSKED.EXE W", the bracketed C will

be replaced with W.  This allows a different

set of batch files to be utilized with SatSked,

giving you a way to have two setups, say one

for dos, and one for windows.  If no parameter

is present on the command line, UO-22.BAT would

be called.

The other bracketed parameters only are sent to

the batch file if the STRTPBPG flag is set to 0

for the appropriate satellite in SATSKED.PRI. In

this case, SatSked expects you to start your own

ground communications software in your batch

file.  The parameters are present for your use

in ending the pass and returning to SatSked.

The parameter X will be A for an abort time, L

for a normal LOS time, I for abort when Idle,

or N (for none) if you have manually started a

pass with the <S> key in SatSked.  The

local_end_time is the Gregorian formatted

local time that SatSked has determined that

the pass should end.  unix_style_time is that

same ending time in units of seconds since

January 1, 1970.  inject_i_time is the number

of seconds after a pass has started to inject

an "I" keystroke, which will satisfy the

EXITAFTER feature of PB.EXE.  The maxPgCycles

parameter is a copy of that entry in

SATSKED.CFG, and is intended to be used to

count how many times PG.EXE should cycle while

trying to upload a file to the satellite.  The

abortWhenIdle parameter is the number of

seconds after PB has obtained an up to date

directory, and is idle, to abort and go back

to SatSked.  This will only happen if an equal

or lesser priority satellite's pass is

conflicting. The next_aosTime parameter is used

to prevent SatSked from abortingWhenIdle until

the next satellite to be tracked is above the

horizon.

[STRTPBPG X local_end_time unix_style_time inject_i_time maxPgCycles

abortIdleSeconds next_aosTime]

This line is present if the STRTPBPG column of

SATSKED.PRI is set to 1 for the appropriate

satellite.  STRTPBPG.EXE starts the TSR

STUFFIT.COM, which prepares a timed keystroke.

This keystroke is what will prevent PB or PG

from "hanging" if no satellite down link packets

are ever heard.

Next STRTPBPG.EXE runs PG, if there exists an

.OUT,.PUL or .XUL file in your satellite

directory.  This is described in detail

below.

Note that the parameters X through maxPgCycles

have the same meaning as described above.

if errorlevel 1 goto no_pb      To conserve cpu memory when running PB, a

batch file will be created by STRTPBPG.EXE,

which calls PB.  If the S key is hit while

STRTPBPG is running, or the LOS time or abort

time is exceeded STRTPBPG will exit with an

error level of 1.  In this case, SPB.BAT is

skipped, and PB is not run.

call spb.bat                    As mentioned above, to conserve memory SPB.BAT

is called to start the PB program.  This saves

several tens of kilobytes which are required

by STRTPBPG.

:no_pb                          Executions branches here if PB.EXE is not

to run.

call ssclean[C].bat             This batch file can be used to terminate

the antenna / tuner tracker, clean out

old .DLX messages, call postpass.bat or

whatever you want to do after each pass.

A master ssclean.bat can be located anywhere

in the searchpath.  You may also override this

master with a local copy in any of the

satellite directories.  This allows you to do

a different type of cleanup for each satellite.

The bracketed C character is replaced with

SatSked's command line parameter if present,

and is omitted if not.

*****************************************************************************

*****SPB.BAT************************Comments:*********************************

pb.exe                          This line starts PB.EXE.

stuffit /r                      This line removes stuffit from memory

******************************************************************************

SATELLITE_NAME[C].BAT:  (The bracketed character C is only present, and is set

---------------------    to the first character of the first parameter on

SatSked's command line)

These files should be located in the SatSked directory.

***UO-22.BAT**********************Comments:***********************************

c:

cd \pb\uo22                     This changes the default drive and directory to

the one  where the specific PB.CFG and PG.CFG

exist for UO-22.  You must use the directory

names and drive for your particular setup here.

call RUNUO22.BAT                A batch file that starts your rotor tracking,

radio tuner, and whatever else you need to do

before  PB/PG is run.

******************************************************************************

RUNUO22.BAT, is specific to my system, so I won't include a copy of it here.

Basically I use a program to change my radio's to the proper mode, USB

RX, FM TX for the psk birds, and FM on both for the 9600 bps satellites.  I

set my doppler correction program's base frequency to that of the satellite

uplink and downlink in order to compensate for doppler shift, and finally

change to the proper modem mode using the DSPMODE program for the DSP-12.

Your requirements may vary from mine.

If STRTPBPG is set to 0 in SATSKED.PRI, then 4 parameters are sent to the

SATELLITE_NAME[C].BAT file.  This file will be responsible for running the

ground station software.  See the section on TRACK.BAT for details.

STRTPBPG.EXE:

------------

SRTPBPG.EXE should be located in a directory that is in the dos searchpath.

This will enable execution from any satellite directory without specifying the

exact path.

When TRACK.BAT is called from SS.BAT after SatSked exits, it starts up your

rotors, radio tuner, and changes to the proper satellite directory through

the call to UO-22.BAT, or another satellite specific batch file.  Next,

LOS or Abort time information is passed to STRTPBPG.EXE.

STRTPBPG searches the satellite directory for .OUT and .XUL files.  If it

finds one, it runs PG.EXE -U -Q. If not, it starts PB.EXE as described below.

To avoid getting stuck in PG.EXE if the satellite never comes, or a slot

doesn't open, STUFFIT.COM is called before PG.EXE.  Depending on the commands

from SatSked, STUFFIT will "press" the Esc key at the ABORT or LOS time.  This

will return control from PG.EXE to STRTPBPG.EXE.

If a .XUL file is found from a reject of an upload, it will be renamed with the

.OUT extension.  If more files exist to be uploaded, STUFFIT.COM will be

reinitialized and PG.EXE will run again.  This process will repeat

"maxPgCycles" times, or until the LOS or ABORT time is reached.  This feature

must be used carefully, and maxPgCycles should not be set too high.

maxPgCycles is set in SATSKED.CFG, and defaults to the value 5.

If there is time left before LOS, SPB.BAT is created, then is called to start

PB.EXE.  Also, STUFFIT.COM is called to press the "I" key or "Q" key from

within PB.  If no abort time is present, "I" will be pressed after the

parameter injectI seconds after PB.EXE is run.  This will satisfy the

requirement that packets are heard by PB.EXE, so PB's exitafter command will

function.  If an ABORT time was present when STRTPBPG was started, the "Q" key

will be pressed at the proper time so SatSked will regain control.

The default action of STRTPBPG may be modified by pressing the S key to

return to SatSked, or pressing the B key to jump directly to PB.

STUFFIT.COM:

-----------

STUFFIT.COM should be located in a directory that is in the dos searchpath.

This will enable execution from any satellite directory without specifying the

exact path.

The program  STUFFIT.COM, is a "TSR", or "terminate and  stay resident" program

that "presses keystrokes at a predetermined time.  If a keystroke is not hit,

and no packets are heard, PB will never exit regardless of the EXITAFTER value

that is set in PB.CFG.  I use the value "1" for EXITAFTER in my PB.CFG file.

PG.EXE has no exitafter command, this is why the Esc key is pressed at the

ABORT or LOS time.  If you run stuffit with a /? parameter, a short usage

statement appears.  One feature I've found useful is the ability to execute

commands from a disk file with the /f:filename switch.  This way several

keystroke events can be stacked up in time.  This could be useful when playing

around with FO-20 or another "non-standard" satellite.  For more information,

see STUFFIT.DOC.

Note that STUFFIT counts time through clock ticks.  Unfortunately, if you are

running DesqView, or Microsoft Windows, clock ticks are "stolen" to swap from

task to task.  This means that STUFFIT won't get 18.2 clock ticks per second,

and it's internal time will run slow.  This results in the injected keystroke

coming later than desired.

TRACK.BAT Examples created by SatSked:

----------------------------------------

Here is an example of a track.bat that was generated for UO-22.  Here I have

started SatSked with a parameter "w" (SATSKED.EXE W).  The STRTPBPG column

in SATSKED.PRI was set to 1, and this is a normal pass with an LOS local time

of 14:30:24:

call  uo-22w.bat

strtpbpg L 14:30:24 737058594 80 10 5 737061111

if errorlevel 1 goto no_pb

call spb.bat

:no_pb

call sscleanw.bat

Here is track.bat which was started manually with the S key.  No command line

parameter was entered when I ran SatSked.  Also the STRTPBPG parameter was

set to 0, so lo-19.bat is responsible to start the ground communications

software, start stuffit etc.

call lo-19.bat N 00:00:00 737109278 80 10 5 73719501

if errorlevel 1 goto no_pb

call spb.bat

:no_pb

call ssclean.bat

Here is track.bat with an Abort time of 14:48:00 local.  The abort function

is flagged with the "A" parameter.

call  ao-16.bat

strtpbpg A 14:48:00 737059650 80 10 5 737059812

if errorlevel 1 goto no_pb

call spb.bat

:no_pb

call ssclean.bat

Here is track.bat which was generated when a conflicting pass of equal or

lesser priority exists.  The "I" parameter tells STRTPBPG to start stuffit so

that it will press the "Q" key when the up to date, idle state is reached in PB.

Note that SatSked will cycle back to the same satellite if the AOS time of the

conflicting bird has not yet been reached.

call io-26.bat

strtpbpg I 23:28:16 753856069 80 10 5 753856350

if errorlevel 1 goto no_pb

call spb.bat

:no_pb

call ssclean.bat

ABORT Times:

-----------

If SatSked has determined that there is a conflict with the subsequent

satellite pass which has a higher priority, STUFFIT.COM is called (through

STRTPBPG.EXE) with the parameters  =HH:MM:SS 'Q', where HH:MM:SS is the AOS

time of the next satellite.  This quits PB, saves all data downloaded to disk.

When we get back to SatSked, the program notices that the current time is

beyond the "abort time" and it moves on to the next pass.  Now the process

repeats with the next satellite.  The level of searches done to check priority

is equal to the number of "priorities" loaded from the file SATSKED.PRI to be

described later.  To abort, the next satellite must have a HIGHER priority than

the first.

If the parameter abortEarly is set to a non zero value, then the abort time

written to sort.dmp will be modified by the number of seconds specified.  To

abort earlier than the AOS time of the higher priority satellite, use a

positive number.

Idle Tasks:

----------

In between satellite passes, it is possible to run an external program, and

use stuffit.com to exit that program in time to run the next satellite pass

from SatSked.  This is done by selecting a non zero value for the SATSKED.CFG

parameter idleTaskRunSeconds.  This parameter is the minimum number of seconds

from the current time, to the start of the next pass, which will allow the

idle task to be started.  If the length of time is shorter than that specified,

the idle task sequence will not be initiated.

SatSked will then create TRACK.BAT, which contains a call the batch file

IDLETSK.BAT, and then exit.  To facilitate returning to SatSked in time to

start the next satellite pass, several parameters are fed to IDLETSK.BAT.

The format is:

CALL IDLETSK[C].BAT +T1 T2 T3 SATELLITE_NAME

where

C is the first parameter when SatSked is started, such as "SATSKED.EXE W",

the bracketed C will be replaced with W.  If no parameter is entered,

the IDLETSK.BAT will appear.  Note that the brackets do not appear.

+T1 is the offset time in hours:minutes:seconds until the next pass.  This

string is properly formated to be used as a direct parameter to

STUFFIT.COM to delay execution for that length of time.

T2 is the absolute local time in hours:minutes:seconds until the next pass.

T3 is the number of seconds until the next pass.

SATELLITE_NAME is the ASCII root name of the batch file which will be

called when SatSked is restarted at the proper time.

Example (KO-23 is due in 11 minutes and 52 seconds at 00:54:06 local dos time),

here is the contents of TRACK.BAT:

call idletsk.bat +00:11:52 00:54:06 712 ko-23

Note that the times are earlier than the actual satellite's AOS by beforeAOS

seconds, as set in SATSKED.CFG.

Here is an example of IDLETSK.BAT that you might use to run TCPIP between

satellite passes, and use stuffit to type "exit" to leave the program:

stuffit %1 'exit' 13

net.exe

stuffit /r

Note: W5RKN advises that this sequence will remove you from "NOS" net:

stuffit +20 13 +5 13 +5 13 %1 @68 +5 'exit' 13

net

stuffit /r

Another possibility is if you want to use your own screen saver, such as

RAIN.EXE, which uses the escape key to exit:

stuffit %1 27

rain.exe

stuffit /r

If you wish to start the Idle Task manually, use the "j" key.

SatSked's Configuration, And How To Change It:

---------------------------------------------

The file SATSKED.CFG contains a detailed description of configurable

parameters, and their default values.  This can be edited before you start

SatSked, or from within SatSked by pressing the "C" key.  This will drop

you into the default text editor TED.COM, and load SATSKED.CFG.  You can

change any parameter by typing it's name followed by your choice.  When you exit

the editor, SatSked will scan this file and update it's parameters to those in

SATSKED.CFG.  If you comment out a parameter with a ";" at the beginning of

the line, it's value will not change back to the default until SatSked is

exited, and re-initialized.

Setting Priorities For Each Satellite in SATSKED.PRI:

----------------------------------------------------

SatSked goes through a "bumping" procedure when 2 or more satellites are

above the horizon at the same time.  The satellite that gets bumped is the

one with the lower priority.  These priorities are set by editing the file

SATSKED.PRI.  This too can be reached from within SatSked by pressing

the "P" key.  When you do this, and exit the editor, SatSked will recalculate

all "abort times" in the program.  If you had previously manually changed

the "abort times" in the  SORT.DMP file (described next), these changes

will be lost.  Higher numbers indicate a higher priority.  The bumped pass is

still displayed, and will have an "abort time" entry in the pass window.

Parameter fields in SATSKED.PRI:

--------------------------------

The first column is the satellite's ASCII name.  This should match the name

in the keplerian elements if automatic AOS/LOS time generation is desired.

Character case is not important.  If there are spaces in the name in the

keplerian elements file, use quotes as shown below.  Note that the spaces

will be replaced with the tilde character (~) when displayed in the AOS/LOS

window, and when track.bat is generated for this satellite.  The satellite

priority is the next column, and valid values range from 0 to 9.  If there

is a 0 in this column, a pass will be not be started.  However, the beeper

will run to let you know that the satellite is present.

The next parameter is the maximum number of runs for that satellite.  The

parameter maxRuns is the value "maximumRuns" in SATSKED.CFG. This tells SatSked

how many times to rerun this particular pass if control is returned to SatSked

before the satellite falls below the horizon.

STRTPBPG is set to 1 if you desire to use the STRTPBPG.EXE program that I have

supplied to start your PB and PG programs (recommended).

Next is the path to the upload directory for messages to users on this

satellite.  If the upload path is not present, you will not be allowed to send

a message to this bird from the message sender (that makes sense huh?).  This

is useful if you want to manually start an event, like your starting your

antennas tracking a voice satellite, such as the RAO13 entry in the example.

An example of SATSKED.PRI is shown below:

;name           pri  maxRuns  STRTPBPG  message path

;----           ---  -------  --------  --------------

UO-22           2       3       1       e:\ham\pb\uo22

KO-23           3       3       1       e:\ham\pb\ko23

AO-16           1       3       1       e:\ham\pb\ao16

lo-19           0       3       1       e:\ham\pb\lo19

fo-20           0       1       0

"Courier 1B r"  0       1       0

"noaa 10"       0       1       0

Automatic AOS / LOS table generation:

------------------------------------

Thanks to Jesse Buckwalter NZ3F, SatSked generates it's own AOS / LOS

times.  When using a math co-processor, this happens very quickly at

program start up.  If you don't have a co-processor, it may take several

minutes to generate new passes.  If the table (sort.dmp) contains at least

one pass, then pass table generation will stop in time to execute that

pass.  If you're starting SatSked for the first time, then make your

kepSearchHours setting small to make sure you quickly get a calculation

for each satellite.  If the program exits because no passes are calculated,

increase the setting of kepSearchHours long enough for a bird to found

crossing the horizon.

If you increase the minimum step rate much past 15 to speed up the

calculations,  you might miss the time that the bird crosses the horizon.

If the horizon has been crossed, and the magnitude of the elevation is

found to be greater than the parameter aoslosAccuracy then the cpu's bell will

sound.

For AOS errors, I back up by 0.2 of an orbit and start going forward again.

I think this works pretty well.  So one beep for AOS is probably ok.  You

can hit the pause key and look at the times listed to see if it recovered.

For LOS errors, I back up the time to the previous value where the satellite

was above the horizon, and reduce the step time to 3/4 of the configured

step time.  This reduction in step time continues until the step time is 1

second.

SatSked allows you to change the elevation at which it considers to be the AOS

or LOS crossing.  This parameter is called genKepMinElevation, and is entered

in degrees positive to delay AOS and accelerate LOS.

If you are not using a co-processor, then the pass generation will seem

pretty sluggish.  Jesse and I have been messing with the algorithm to speed

this up, but have run into problems.  Jesse suggests we stick with this one

for now as it appears to be accurate.

When SatSked starts, it looks at the number of passes loaded.  If this number

is less than the parameter "sortGenThreshold", SatSked will start adding to

the pass table automatically for each bird in SATSKED.PRI which has a non

zero priority, or all of them if "genKepZeroPriority" is set to 1.  In this

case, the name must match one of the names in the keplerian element file.

While the pass generator is chugging away, it is watching for the time of

the next pass.  If this time is exceeded, it gives up and discards that

partial pass, beeps, creates track.bat and exits as usual.  Thus, even though

it's slow, you should never miss a pass due to slow calculations.  If you

happen to be at the CRT during this, and want to quit, just hit a key.  The

last set of passes completed will be added to your pass table (sort.dmp).  Note

that some of the birds may not have been gotten to, this is ok.  The next time

the number of passes drops below "sortGenThreshold", and you leave the

keyboard alone, SatSked will pick up where it left off for all birds, and

compute to "kepSearchHours" past the last valid bird in the table (any bird)

for all satellites.

You may want to play with these two parameters to make this convenient for

your particular operation.  Note that entries you have manually edited in the

pass table won't be affected, unless n passes near the end of the sort.dmp

file.  N is the number of non zero priority satellites in SATSKED.PRI, or

the total number of birds in that file if "genKepZeroPriority" is set.

Additional passes may be generated by pressing <G> then selecting <1> for

"append to present".  If you wish to replace all the passes, select option

<2>.

Manually Changing AOS, LOS and Abort Times in the SORT.DMP file:

---------------------------------------------------------------

When the priorities have been scanned, the "Abort" times are stored in the

last column of the SORT.DMP file.  If you wish to change these times, or

any of the AOS or LOS times, or delete a pass entirely, you may edit

SORT.DMP.  Once again, this can be done from within SatSked by pressing

the "E" key.

Be careful what you edit!  SatSked expects it's times at the proper column.

When you exit from the editor, SatSked will immediately use the new times you

entered.

Keplerian Window:

----------------

Up to 100 satellites may be tracked by SatSked.  Each of the satellites in

the keplerian data file may be displayed.  In the normal display mode,

the satellites are displayed as they are found in the file KEPORDER.TXT.  This

file should contain just the name of the satellite to be displayed, one bird

per line.  Following the name of the bird is an optional parameter which will

cause the AOSLOS display field to be blanked out for an entry of 1, or replaced

by the satellites altitude in meters if the number 2 is entered.  This can

be useful to avoid the long computation times necessary to find the horizon

crossing time of individual satellites.  Edit that file with a text editor to

change the display order.

The size of the real time tracking window is determined by what's left over

after the pass window size is specified with passesToDisplay.  The delta E

indicates the direction of change of the elevation by an arrow up or down,

and the dR/dT is the rate of change of the range from you to the satellite

in Kilometers / Second.  This is used for doppler calculations.  Use the

control key with the up arrows, page up, page down, control and home keys to

scroll through the satellites.

If displayAOSLOS is set to 1, the rightmost column will may out the hours,

minutes, and seconds to LOS or AOS of that satellite.  Performing this

floating point calculation is VERY cpu intensive, and I don't recommend it

unless you have a coprocessor.  However, the calculations only are made once

for each horizon crossing.  The values are saved on disk when SatSked exits,

and restored when the program starts back up.  When paging through the

keplerian window with control arrow/home/end/page keys, there will be a

delay when any satellite that was not present in that window suddenly appears.

If horizonBeep is set to 1, then SatSked will send a rising or falling tone

to the CPU speaker.  This will occur even with the screenSaver, but the time

of occurance may be off by as much as 30 seconds or so.  This is due to the

fact that the position of one satellite per second is computed while the

screen saver is operating.

OrbitDRV Window:

---------------

This window will appear if SatSked detects the presence of OrbitDRV or RotorDRV.

Both of these programs are TSR's that are used to control rotor movement.  There

are two lines of information displayed, here is a list of the display items:

1.  Ta and Ca are target and current azimuth,

2.  Te and Ce are target and current elevation.

3.  A indicates bad station elements.

4.  E indicates bad satellite elements.

5.  R indicates that RotorDRV has been enabled,

6.  T indicates that OrbitDRV's tracking is enabled.

7.  U indicates that OrbitDRV's tuning is enabled.

8.  D indicates that RadioDRV is being driven directly every second from SatSked

without the intervention of OrbitDRV.

9.  I indicates that the KCT software has detected an I/O error with the KCT

board.

10. S next to Ca or Ce indicate that the rotor has stalled.

11. The arrows indicate the direction of rotor movement.  Note that they ae

sampled once a second, and small movements may not be indicated.

12. The name of the satellite tracked will appear as no data if either the

station elements are bad, or the satellite elements are bad, and

SatSked is not driving RotorDRV direct.  The bird name is shown

otherwise.

KCT Tuner Issues:

-----------------

I have removed all code pertaining to the dummy satellite which was used

previously to assure that the Kansas City Tracker code will correctly

initiate tuning.  The problem was found to be that a period of time must

elapse between disabling the KCT, and reloading it with new elements. This

time period can be set with the KCTdisableDelay parameter.  A value of

2200 (milliseconds) is recommended.

Site Longitude Entry:

--------------------

Jesse NZ3F wrote the orbital tracking code which I adapted to use inside

SatSked.  I've found that longitudes west of longitude 0 (England) should be

entered as a negative number.  Longitudes east of longitude 0 should be

entered as a positive number.  This appears to match the way InstantTrack works.

Message Sending Utility:

-----------------------

To create a message to be uploaded to a specific satellite, press the "M" key

from SatSked.  You will be prompted for the destination ham call, title,

keywords, expiration time, and optionally the message filename.  If

useTimeMsgName is set to 1, the filename will be the current time with the

extension .MSG.

Next you will be placed in the editor specified by the parameter "editor" in

SATSKED.CFG.  The source and destination ham calls will be entered into the

file, along with the time and date.  This message header may be removed with

the "addMessageHeader" parameter in SATSKED.CFG set to zero.  I personally

hope you don't exercise this option as my hard disk will fill up with messages

that I don't know the sender, recipient or time of creation.  If you're worried

about the room it takes, compress the file or delete your signature!

If the destination call exists in the file USERLIST.TXT, (renamed from

UO22USER de JA6FTL) which you should locate in your SatSked directory, the

corresponding ham's name will be inserted in the message file for you.  To the

bottom of your message will be appended the file "suffixFile" as

set in SATSKED.CFG.

When you exit, you will be given the option of sending along additional files

with the text file you just created.  You may enter a path to the desired

drive and directory where the file to upload is located.  Then enter the

filename from the displayed directory, wildcards may be utilized with

the filename.  Note, you will not be able to send extra files if you

have selected no compression as your default "compressor".  Also I've added

the mask feature to select display of files that match a certain

specification.

You must have all the files you want to send in one directory.  So you

can append just one file, or a group of files with a wildcard.  If you

have a group of unrelated files to send, place them in an otherwise

empty directory by shelling to dos first before you send the message.

The directories where you would like the xxxxxx.out files to be sent when

creating a message is specified by the last parameter in SATSKED.PRI, and is

described above.  The directory should contain the complete path, and no

trailing \ character. Note that the message sender will not work if the path

is empty for that satellite.

If it is desired to save the text part of each outgoing messages, the

"msgDir" parameter should be specified in the SATSKED.CFG file.  All

outgoing messages will be copied to this directory.

Message Compression:

-------------------

Presently SatSked's messages can be sent uncompressed (default), zipped (zip)

or lharc'ed (lzh).  Also you can specify "auto" which will try all three, and

send the smallest resultant file.  If the source filesize is less than

"compressionThreshold" bytes long, the uncompressed version will be sent

regardless.  The idea behind this is to send the smallest file to the

satellite.  If the message is a small one, the convenience of not needing to

decompress it outweighs the savings of a few bytes of "ramdisk" space on the

satellite.  The default compression is 1000 bytes

Dumb Terminal:

-------------

By pressing "T", you will be placed in a basic terminal mode, with the comm

port parameters set in SATSKED.CFG.  You can talk to any port, at any address,

any interrupt number, and any baud rate.  These parameters are set by

commPort, cpAddress, cpInterrupt, and baudRate respectively.  Also tab

characters may be expanded by setting expandTabs to the desired length.

If the parameter addUploadLF is set to 1, a LF character will be added to

the carriage return when each line is uploaded.  This is the way that

SatSked versions up to 1.7 operate.  Now this parameter now defaults to 0, and

no linefeed character will be uploaded.  This should fix a problem when you

try to upload keplerian elements to the TrakBox.

You may log any incoming text to disk by pressing F3, upload an ASCII file

with F2 and return to SatSked with F1.  When you upload a file, the parameter

ASCIITransferDelay delays transmission by N milliseconds after each carriage

return is sent.

SatSked has a clock in the lower left hand corner. This clock will

flash if SatSked would have met the criteria to beep and indicate that the

current pass is ready to start when you leave the dumb terminal with F1.

There are two programmable function keys, F4 and F5.  These keys may

contain up to 40 characters, and are specified by the desired text in

quotes  following the parameter name termF4key and termF5key in SATSKED.CFG.

Trakbox Support:

---------------

A second dumb terminal has been added to SatSked.  The intention is for this

terminal to be used with the TrakBox comm port if you have one.  The setup

parameters for this terminal begin with tb, and are functionally the same as,

but are separate from the other terminal parameters.  Use control T to access

the trakbox dumb terminal.

If tbTrackOnExit is set to 1, the same comm port as configured above will be

used to start the TrakBox in it's single track mode.  SatSked sends an ascii

"q" to the TrakBox, to return to the main menu, then then selects option 1,

Real Time Tracking.  SatSked watches the response from the start of this menu

for the position of the desired satellite to track.  If that bird is present,

SatSked selects the proper position.  If not, the TrakBox is sent back to the

main menu, and the TrakBox start tracking function is aborted.  SatSked will

present an error message, and continue on with the pass.

If the parameter tbupdateClockOnExit is set to 1, then SatSked will use option

5 to set the TrakBox clock to UTC time, from the dos clock.  This function

may also be performed at any time manually by pressing "h".

If the paramter tbAutoKepUpdate is set to 1, when a new keplerian element

file is found, the new keplerian elements are uploaded to the TrakBox.  The

frequency information file will then be uploaded automatically.  For this to

occur, the file containg frequency information and position, FREQ.DAT,

must be located in the SatSked directory.  This function may also be performed

at any time manually by pressing the "z" key.  Be careful with this option!

If a garbage keplerian element file is chosen automatically, the TrakBox's

data base could be lost, resulting in subsequent lost satellite passes.

If you wish to manually start the TrakBox tracking a satellite, press the

control and "l" keys simultaneously.  The enter the exact satellite name and

hit return.  Note that this satellite must be found in the first 16 positions

of the TrakBox, or the function aborts.

SatSked Keys:

------------

There are now five menu pages that are accessed by hitting the Tab key to

go forward, and the shift-Tab key to go backwards.  All of the functions

listed in the menu are alive outside of that menu page.  In other words you

don't have to see it to do it.  SatSked now saves the active page before

exiting, and restores it upon program startup.

Now there are 12 user command keys (F1-F12), along with a configurable dos <D>

and view-dir <V> key, for a total of 14 user defined functions.  D is intended

to be used for shelling to dos, and V for View-Dir by ZL2TPO.  You of course

can set these to whatever you want with the "dosCommand" parameter, and the

vCommand parameters.  Enclose the userCommand in quotes if you wish to send

a parameter to the desired program:

example:    userCommand "q.exe textfile.txt"

If you don't use all the userCommand keys, F12 and those unused below that

key will shell to dos.

When a pass is after the time set by "beepTime", the pending pass time

flashes on the screen.  A screen saver will run if no keystrokes are heard

after the time limit you set.  The time to the next pass is still displayed, and

"walks" around the screen.

You may page through the "pass window" to examine future passes with the

usual cursor keys.  Also, any conflicting passes are colored differently, so

you can easily notice them and change the abort time if  you want.

Daily Batch file:

----------------

If desired, SatSked will run a specified batch file once per day at a user

specified time.  The start time should be specified with the "dailyBatchTime"

parameter, and the approximate length of time that this file will take

should be specified with the "dailyBatchRunSeconds" parameter.  The name

of the batch file that is run is fixed, and is called DAILY.BAT.

SatSked Operation with DesqView:

-------------------------------

SatSked may be "tuned" for minimum cpu cycles when used with DesqView.  The

parameter "dvSwaps" may be adjusted to allow DesqView to regain control when

SatSked is waiting for a keystroke or a pass to start.  Values from 1 to

32767 are valid.

Dos Clock Error Correction:

--------------------------

Typically, the hardware clock on your motherboard is more accurate than the

dos clock (at least mine is).  The dos clock may also lose time if software

timer tick interrupts are lost due to buggy TSR's, desqView, Microsoft Windows,

etc.  SatSked can be configured to periodically read the hardware clock, and

set the dos clock to that value.  In addition, SatSked can adjust the

hardware/dos clock by a daily offset number of seconds, to correct for a

known drift.  Note that this offset may be positive, or negative.  The offset

parameter is clockAdjustSeconds, and the parameter to periodically update the

dos clock is residentClockMinutes, which is specified in minutes between

updates.

If it is desired to only update the dos clock when SatSked starts a pass, then

set the parameter updateClockOnExit to 1.  The daily offset will still be

utilized, as long as SatSked starts a pass at least once a day.

The way the offsets are handled are as follows:  To achieve n seconds per day

correction, the day is broken down into n segments of time.  When each time

segment has passed, a second is added (or subtracted if negative offset)

from the dos clock.  If the exact number of segment seconds has occured, as

is not usually the case, the number of seconds which should have been adjusted

is issued, and the "effective" last adjustment time is saved to disk for the

next update.

As an example, lets say clockAdjustSeconds is set to 24.  This means every hour

the dos clock should be offset by +1 seconds.  If 2.5 hours elapses between

adjustments, due to no satellite passes until that time, or if

residentClockMinutes is set to a high value, then the clock is offset by +2.

Then, SatSked calculates the the time of adjustment back by a half hour, and

saves this time to disk.  If a half hour goes by and an adjustment is made,

it will appear as if it's been an hour since the last adjustment, and the

clock is again updated by +1 second.

If more than 3 days elapse between adjustments, such as if you shut off your

computer for a few days, no daily offset will be made to compensate for that

off period.  The "offset timer" will start over from scratch.  Thus you must

accurately set the clock if the CPU's been powered down for a long period of

time.  This is done because the hardware clock will drift differently at a

lower temperature.

If your clock is in really bad shape, you can cause SatSked to update the clock

at regular intervals.  Set the parameter residentClockMinutes to the desired

number of minutes between updates.

I've had pretty good success with this method, easily keeping the dos clock

within a second a day.  It's not automatic, you have to keep an eye on it, but

the price is right!

Timezones and SatSked:

---------------------

SatSked determines the proper timezone from the dos environment variable TZ.

This is also used by InstantTrack.  If TZ is not set, it will default to

Eastern Standard Time, which is UTC - 5 hours.  Here are some examples of

what you might put in your autoexec.bat file:

SET TZ=EST5EDT		Eastern Standard Time (default)

SET TZ=CST6CDT          Central Standard Time

SET TZ=PST8PDT          California Time

SET TZ=UTC0             England, or if you use UTC on your dos clock.

SET TZ=AUS-10           VK3 stations.

You can eliminate the string of characters after the offset number, and SatSked

will not revert to daylight savings time.  The time of year that the daylight

savings time switch is made is fixed, and is either set by DOS or the compiler,

I admit to not knowing which.  I do know that you must set your country code

in your DOS CONFIG.SYS file.  This controls if DOS considers summer to be in the

July timeframe, or January.

The offset from UTC in hours that SatSked is currently using is displayed at

the top left of the SatSked screen, mine says UTC-5 for non daylight savings

time.  All pass data in the window will be adjusted to this timezone.

Silent Operation:

----------------

By hitting the "I" key, all sounds from SatSked can be enabled, or disabled.

This will also disable/enable the AOS beeper, in all modes.  The state of the

sound will be saved when SatSked exits.  No longer do you have to listen to

beeps and boops from SatSked all night, just punch the I key before you go to

bed.

Bells and Whistles:

------------------

You can change the colors of the program, change the parameters which

determine when a pass is started, or blocked, or change the number of times a

single pass will be attempted before giving up.

You may also change the sound of the beeper, use a morse code annunciator

to spell out the satellite to be tracked, or make SatSked call a "beep batch

file" at the pending pass beeptime.  Also you can make the beeper function

operate or not on priority zero satellite passes (these will not generate

track.bat and cause SatSked to exit).  With SatSked 1.931, the beeper will only

actuate the first time a particular pass is about to start.  On subsequent

passes, the beeper won't sound.

Some Other Ideas:

----------------

You could edit the SORT.DMP file and create your own fake satellite pass,

setting the name to one which will call a special batch file.  This file could

update your dos clock at a particular time of the day, ring your phone at

work to let you know that your house hasn't burned down from sticking rotors,

or close a relay and shock your dogs!  Make sure that you place the appropriate

maximum runs entry in your SATSKED.PRI file for that event (usually set to 1).

Comments:

--------

SatSked was written using Borland C++ version 3.1, which I recommend

highly.  It was compiled for the generic 8086 processor, and will use

a coprocessor if present.

Please report any bugs to me promptly.  Include the offending portion of

SORT.DMP, a description of the error, the time it occurred, and anything

else that you think might help to fix it.  I can be reached most reliably on

Compuserve, account number 73055,1733.

Thanks:

------

To all my beta testers, who have helped me develop this program throughout

1993, I wish to extend my sincerest gratitude.  Without you, SatSked wouldn't

be half the program it is now.  Thanks for all the ideas you've contributed,

and the help you gave me sorting out the bugs.

Special thanks to Frank Viggiano, KB2MVN for the long hours and many dollars

spent on the telephone with me, downloading my bug ridden code and subjecting

his computer to the resultant countless crashes.  Also I must thank him

for all my new grey hairs, due to his relentless requests for new features.

I only hope his poor XYL, Carla, will decide to keep him after all the time

he's spent hiding in his shack!

Good  luck tracking!

73's  de  WA2N,

Wayne D. Roth.

Appendix A:

----------

Obtaining Schedule Data From QuikTrak For Use By SatSked:

--------------------------------------------------------

SatSked uses an ASCII file containing a list of satellite AOS's and LOS's.

This file is generated by QuikTrak's "SCHEDULE" function.  SCHEDULE is run

from within QuikTrak for each satellite of interest.  Data is saved to a

single disk file using the "f" option when you're prompted for the start

date.  Several satellites can be saved to the same file.  Each time you run

the Schedule function, it will add to the previous data in that file. You may

generate as many "passes" as you wish, but SatSked will discard those passes

beyond the 400'th.  When you have written all the pass data to the default

filename SCHEDULE.DAT, exit from Quiktrak.

Now sort the passes chronologically by running the program QTSORT.EXE with

the switch /d.  Qtsort was written by KF5OJ, and is included with SatSked.

The output filename will default to SORT.DMP.  This is the input file that

SatSked uses.  Copy this file to C:\SATSKED\SORT.DMP.  SatSked cannot find

SORT.DMP unless it's in the same directory as SATSKED.EXE. SatSked will abort

if this file is not found, is empty, or contains old data.

Obtaining Schedule Data From ITPASS.EXE:

---------------------------------------

ITPASS.EXE interrogates InstantTrack's database IT.ORB and extracts schedule

information for the desired satellites.  This schedule information is then

translated by the program ITPSCONV.EXE to the file SCHEDULE.DMP.  Now Sort

the passes with QTSORT.EXE as outlined above.  Here is a batch file that will

perform all the steps necessary for satellites AO-16, UO-22 and KO-23:

******ITPSSORT.BAT*************************************************************

ITPASS -d -n 30 AO-16 UO-22 KO-23 > ITPASS.DAT

itpsconv.exe

qtsort /d

*******************************************************************************