SUBJECT:  N2GTE'S SPREAD SERVER (c) 1990 Doug Miller N2GTE

Documentation Written by KA3DXX [George Stickler]

Partially Approved by N2GTE [Doug Miller] (up through the end

of Part I).

Part I - Setup for the N2GTE Packet Message Switch:

1.  Files Required:

a.  SPREAD.COM and SPREAD.CNF.

b.  Appropriate entries in the \FWD. file and in the \CHORE.xx file.

EXPORT the INPUT file information via the \FWD. file and IMPORT the

OUTPUT file information via the \CHORE.xx files.

2.  SPREAD.CNF File Layout:

C:\GTE:\SPREAD.IN        <-- Drive:\path\filename of the INPUT FILE.

C:\GTE:\SPREAD.OUT       <-- Drive:\path\filename of the OUTPUT FILE.

C:\GTE:\PUBLIC\ASCII\    <-- Drive:\path\dirname where *.DIS files reside.

NOTE:

Since the SPREAD Server has a configuration file, you can put

SPREAD.COM, SPREAD.CNF, and the INPUT and OUTPUT files anywhere

you want to as long as you put the actual (or SUBSTituted)

drive:\path\filenames in the SPREAD.CNF file. I have mine in my

E:\ drive, which just happens to be my root GTEPMS drive.  You can

also put the distribution list files (*.DIS) anywhere you want as

long as you specify the drive:\path\ -- The trailing backslash (\)

IS required.

3.  Use the two fields of the \FWD file as follows:

a.  Enter the DIStribution List Name (less the .DIS suffix) in

the first field.

b.  Enter the Drive:\path\filename of the INPUT FILE in the

second field.

c.  Example \FWD. File Entries:

GTEPMS  C:\GTE\SPREAD.IN

CAMARA  C:\GTE\SPREAD.IN

DCABOD  C:\GTE\SPREAD.IN

BOARD   C:\GTE\SPREAD.IN

TECH    C:\GTE\SPREAD.IN

4.  Use the \CHORE.xx files to IMPORT the OUTPUT file as follows:

C:\GTE\SPREAD.OUT

C:\GTE\REQBBS.OUT

C:\GTE\REQNOD.OUT

5.  Place a "timed" entry in the PORTER.CNF file for the Spread

Server SS-PIF.DVP file, as follows:

T 06 Q:\SS-PIF.DVP CHORE.DUM

NOTE:  The CHORE.DUM entry is a space holder and serves no

purpose other than to provide the parameter that the

PORTER.EXE program file expects to see here.

6.  DISTRIBUTION (*.DIS) Files Layout:

a.  The distribution files for the SPREAD server to work against

consist of the calls and their @BBS calls for the individual Hams

that are to receive the private messages.  An example is the

GTEPMS.DIS file, which follows (as of 17 February 1991):

GTEPMS.DIS:

N2GTE @ N2GTE.MD

WA7NTF @ WA7NTF.MD

KA3DXX @ KA3DXX.MD

WB3V @ WB3V.MD

KA3RFE @ KA3RFE.MD

N3ETI @ N3ETI.MD

WB3FFV @ WB3FFV.MD

WA3MEJ @ WA3MEJ.MD

N3GIY @ N3GIY.MD

GTEDOC @ KA3DXX.MD

b.  CAUTION:  DO NOT FOR ANY REASON INCLUDE A DRIVE:\PATH\FILENAME

ENTRY IN THE DISTRIBUTION FILES -- THAT WILL CAUSE A PROBLEM WITH

THE DV-MAIL.DAT FILE AND CRASH YOUR SYSTEM!  IT HAS HAPPEND TO ME

ALREADY.  I FOUND OUT WHAT "NOT TO DO" - de George, KA3DXX

7.  As an alternative to putting a path in the *.DIS files (see

paragraph 6b above), you should use a DUMMY call (up to six characters,

such as GTEDOC) for the item you want to send to a file and make the

appropriate changes in the \FWD. file and set up the slicer to work

against it in a "timed" batch file or do it manually.

a.  Setting it up in the \FWD. file:

first field     second field

GTEDOC          C:\GTE\GTEDOCS\ANYFWD.TXT -- this is for

anything that goes to GTEPMS.

b.  Batch File Entry:

C:\GTE\SLICEANY\SLICEANY.EXE C:\GTE\GTEDOCS - this will slice

up anything that

goes into the ANYFWD.TXT file in the C:\GTE\GTEDOCS directory.

Part II - Setup for the WA7MBL PBBS Code:

Although KA3DXX wrote this documentation with a slant toward

the GTEPMS code, N2GTE first used the SPREAD Server wih the WA7MBL

PBBS Software, so it is VERY compatible with that software, and can

be used with any PBBS software that will do import/export through

either a Forwarding File, server process, or "on-line" by the SysOp.

With the WA7MBL PBBS software, Doug either used it with the FWD file

or "on-line".  Since I (KA3DXX) also ran the WA7MBL PBBS code, I will

present the non-GTEPMS information from that point of view.

1.  Files Required:

a.  SPREAD.COM and SPREAD.CNF.

b.  FWD.BBS File.

2.  SPREAD.CNF File Layout:

C:\BBS\SPREAD.IN     <-- Drive:\path\filename of the INPUT FILE.

C:\BBS\SPREAD.OUT    <-- Drive:\path\filename of the OUTPUT FILE.

C:\BBS\YFILES\ASCII\ <-- Drive:\path\dirname where *.DIS files reside.

4.  Make the appropriate entries in the FWD.BBS file to EXPORT the

message to the INPUT file (SPREAD.IN) for the MAIL-TO-TEXT TRANSFER;

make the appropriate entries in the FWD.BBS file that will IMPORT the

OUTPUT file (SPREAD.OUT) for the TEXT-TO-MAIL transfer process.

FWD.BBS Entries:

------

X SPREAD.IN         <-- EXPORT TO THE INPUT FILE.

00-23

------

D @C:\BBS\SPREAD    <-- CALL THE SPREAD SERVER.

ANYDIS              <-- FIRST SPREAD DISTRIBUTION

ANYLST              <-- SECOND SPREAD DISTRIBUTION

00-23

------

T SPREAD.OUT        <-- IMPORT TO THE OUTPUT FILE.

00-23

-------

5.  BATCH File Structure:

FILENAME:  SERVER.BAT

C:\

cd \bbs

IF EXIST SPREAD.IN C:\BBS\SPREAD

If you use the BATCH file, you would call it in the FWD.BBS file

instead of the SPREAD SERVER itself, so this is how the entry in

the FWD.BBS file will look:

------

D @C:\BBS\SERVER    <-- CALL THE SERVER BATCH FILE

ANYDIS              <-- FIRST SPREAD DISTRIBUTION

ANYLST              <-- SECOND SPREAD DISTRIBUTION

00-23

------

Part III - AA4RE Setup for N2GTE's SPREAD Server:

Although KA3DXX wrote this documentation with a slant toward

the GTEPMS code, N2GTE first used the SPREAD Server wih the WA7MBL

PBBS Software, so it is VERY compatible with that software, and can

be used with any PBBS software that will do import/export through

either a Forwarding File, server process, or "on-line" by the SysOp.

Since N3ETI ran both the AA4RE and WA7MBL PBBS software, he will

present the information necessary to run the SPREAD Server with the

AA4RE PBBS System.

=======================