TNT - A UNIX Packet-Radio Terminal Program

------------------------------------------

(last modified: 94/03/17)

Index

-----

0. Introduction

0.1 License, copying, warranty

0.2 About Packet Radio and TNT

0.3 Why this program?

0.4 What's needed?

1. Description

1.1 Virtual screens

1.2 Main statusline

1.3 Keyboard-commands

1.4 Commands in command mode

1.4.1 TNC-commands

1.4.2 External commands

1.4.2.1 Writing data to files

1.4.2.2 Sending files

1.4.2.3 Shell and redirect

1.4.2.4 Directory

1.4.2.5 Miscellaneous

2. Special features

2.1 Remote commands

2.2 Umlaut conversion

2.2.1 Display

2.2.2 File-receive

2.2.3 File-send

2.3 Shell-login and Redirection

2.4 AutoBIN: File transfer

2.5 Special connect text with macros, Name-database

2.6 Routing scripts

2.7 Call update

2.8 Logbook

3. Installation

3.1 Configuration file

3.1.1 Serial configuration

3.1.2 Security

3.1.3 Directories and Files

3.1.4 Lines of virtual screens

3.1.5 Display configuration

3.1.6 Screen attributes

3.1.6.1 Color attributes

3.1.6.2 Monochrom attributes

3.1.7 Packet assembly timeout

3.2 TNC-configuration files

3.3 Cookie file

3.4 Files for remote commands

3.5 Files for connect text

3.6 Files for names database and routing scripts

3.7 User-Id's and security

4. Options at startup

5. Running under X11

6. Porting of TNT

7. The end

0. Introduction

---------------

0.1 License, copying, warranty

------------------------------

TNT is Copyright (C) 1993 by Mark Wahl, DL4YBG

This program is free software; you can redistribute it and/or modify

it under the terms of the GNU General Public License as published by

the Free Software Foundation;

This program is distributed in the hope that it will be useful,

but WITHOUT ANY WARRANTY; without even the implied warranty of

MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.  See the

GNU General Public License for more details (contained in file 'license').

You should have received a copy of the GNU General Public License

along with this program; if not, write to the Free Software

Foundation, Inc., 675 Mass Ave, Cambridge, MA 02139, USA.

0.2 About Packet Radio and TNT

------------------------------

Packet-Radio is a special mode used by Radio Amateurs to transfer text and

data. It is packet orientend and uses the AX.25-protocol (a special version

of X.25 to fit Ham Radio needs).

There are some single board computers which implement the AX.25-protocol

and contain the modem-interface to the radio-transceiver. The terminal or

computer with terminalprogram is connected via a RS232-interface. These

single board computers are called Terminal Node Controller (TNC).

For most of these TNCs a special software is available (WA8DED-Software or

The Firmware by NORD><LINK) which can be switched to a special protocol

(hostmode) at the RS232-interface. If hostmode is selected it is not

possible to operate the TNC using a simple terminal, a computer with a

special terminalprogram is needed.

The advantage of hostmode is that the terminalprogram can implement virtual

screens for the different connections, the command mode and monitor mode.

All these will be displayed together on a normal terminal which leads to

confusion of the operator.

TNT is such a terminalprogram (TNT stands for TNc Terminalprogram).

0.3 Why this program?

---------------------

There are some implementations of the AX.25-protocol available for UNIX

(KA9Q, WAMPES, ... )

They all use the TNC as a simple modem (KISS-mode) and have TCP/IP protocol

implemented (using AX.25).

But the operator frontend is very poor, there are no virtual screens and

scroll back buffers.

So my intention was to have a program which has a powerful operator frontend

and uses the TNC not as a modem but as a Terminal Node Controller.

0.4 What's needed?

------------------

The program is developed for LINUX and a VT100-compatible terminal. It

further needs a TNC with WA8DED-software or The Firmware by NORD><LINK

connected to a serial port of the computer.

Although LINUX is the development platform a porting to other UNIXes shall

be possible because no special functions are used. Other terminals can

be used if they have at least line insert and line delete and an entry

in /etc/termcap.

1. Description

--------------

1.1 Virtual screens

-------------------

It is possible to operate several connection on the same frequency.

Therefore a virtual screen for every connection is implemented. It is

splitted to a part where all input is typed and a part where all received

data will be displayed. A statusline with all information about the

connection is located between these two parts.

To change parameters or give commands to the TNC a command screen is

available. To simplify operation all commands can be entered in the connect

screens, too by using a colon (:) as the first character.

It is possible to monitor all activity of other stations on the frequency.

To display these information a monitor screen is available.

Because only one real display is available the operator must choose which

of the several virtual screens he wants to see. He can switch between them

by using special codes or keys on the keyboard.

To see which screen and channel is active and to give global information a

statusline is displayed at the bottom of the screen.

All virtual screens (or the two parts of the connect screen) can be

configured larger than the real display on the screen. Therefore only a

small window of the virtual screen is displayed. The window can be moved by

the operator.

1.2 Main statusline

-------------------

The statusline at the bottom of the screen shows the main status of the

program. It shows the screen type (connect, command or monitor) and the

current channel. In addition any open file (send or receive) on the

current channel will be displayed.

In addition a 'P' will be displayed if the data output is paused,

a 'I' if insert mode is active. If the hostmode synchronisation is

lost a 'S' will be displayed. If the TNC was busy and a resend of data

from the computer is done, a 'B' will be displayed.

If data is received on a channel which is not displayed the channel number

will be displayed until it is switched to the connect screen of this channel.

If a connection is active on a channel it is displayed by a '+' at the

channel position.

If a file is open on the displayed channel, a two-character file type

identifier and the filename (only last 14 charcters) will be displayed

in the statusline. Files which receive data will be displayed in the first

file field, files which transmit data will be displayed in the second file

field. The two-character file type identifier will be explained in the

file command chapter.

1.3 Keyboard-commands

---------------------

F1 - F9 : switch to connect screen of channel 1 to 9.

F10     : switch to connect screen of channel 0 (unproto channel).

F11,

cntl-U  : switch to monitor screen,

if in monitor screen switch back to last selected screen.

F12,

cntl-T  : switch to command screen.

cntl-Q  : switch to connect screen of current channel.

cntl-I	: select a new channel, a '?' as the channelnumber in the statusline

indicates this. A number between 0 to 9 for channels 0 to 9 or

'@' for channel 10 must be entered.

cntl-P  : toggle stop of data-output on the current screen,

only possible on connect and monitor screen.

stop is indicated by a 'P' in the statusline at the bottom.

(for monitor screen and received data part of connect screen)

cntl-I	: select a new channel, a '?' as the channelnumber in the statusline

indicates this. A number between 0 to 9 for channels 0 to 9 or

'@' for channel 10 must be entered.

cntl-V  : the pass character. If for example you want to send cntl-T to the

connected station first type cntl-V and then a 'T'. A different

attribute of the character shows that it is a control character.

(for input part of connect screen)

cntl-R  : move the display window of the virtual screen one line up,

if not on top of virtual screen.

(for monitor screen and received data part of connect screen)

cntl-C  : move the display window of the virtual screen one line down,

if not on bottom of virtual screen.

(for monitor screen and received data part of connect screen)

Page up,

cntl-W  : move the display window of the virtual screen one page up,

if not on top of virtual screen.

(for monitor screen and received data part of connect screen)

Page dwn,

cntl-Z  : move the display window of the virtual screen one page down,

if not on bottom of virtual screen.

(for monitor screen and received data part of connect screen)

Arrow key left,

cntl-S  : move the cursor one character to the left, if not on the start of

the line.

(for command screen and input part of connect screen)

Arrow key right,

cntl-D  : move the cursor one character to the right, if not on the end of

the line.

(for command screen and input part of connect screen)

Arrow key up,

cntl-E  : move the cursor one line up, if not on the top of the screen.

(for command screen and input part of connect screen)

Arrow key down,

cntl-X  : move the cursor one line down, if not on the bottom of the screen

(for command screen and input part of connect screen)

cntl-A  : move the cursor to the start of the line.

(for command screen and input part of connect screen)

cntl-F  : move the cursor to the last non-space character of the line.

(for command screen and input part of connect screen)

cntl-Y  : delete all characters from the cursor position to the end of the

line.

(for command screen and input part of connect screen)

cntl-N  : toggle insert-mode. If insert-mode is active a 'I' is displayed

in the statusline at the bottom.

In normal mode all characters are overwritten, in insert mode

all characters from cursor position to the end of the line will

be shifted one position right.

(for command screen and input part of connect screen)

DEL,

cntl-H  : delete character left of cursor and move cursor one position left.

If insert-mode is active, all characters from cursor-position to

the end of the line is moved one position left.

(for command screen and input part of connect screen)

cntl-L  : The character at the cursor position will be deleted, all

characters right of the cursor up to the end of the line will be

shifted one position left.

(for command screen and input part of connect screen)

CR, LF  : The line from the beginning up to the cursor position is

transmitted on the current channel (input part of connect screen)

or will transferred to the command interpreter (command screen).

If the first character of the input line in the input part of the

connect screen is a colon (":"), the line will be sent to the

command interpreter, too (without the colon).

1.4 Commands in command mode

----------------------------

1.4.1 TNC-commands

------------------

Most commands in hostmode consist of one character. It is difficult to

remember the charcter for a not often needed command. Therefore more verbose

command names were created for every TNC command. But it is still possible

to use the original command characters.

Only the known commands are implemented, if you have a TNC software which

has additional commands, the command TNC (see: 1.4.2.5.) must be used.

More information about the commands can be found in the documentation of

the TNC software (TNC commands of NORD><LINK The Firmware 2.6a).

DAMADIS,"B"     : DAMA timeout

VERSION,"V"     : Shows version of TNC-software

CHECK,"@T3"     : Connection timer T3

CONNECT,"C"     : Start connection

CTEXT,"U"       : Response text if connect is received by TNC

DIGIPEAT,"R"    : Digipeat function

DISCONNE,"D"    : End connection

DAYTIME,"K"     : Time and date functions

FRACK,"F"       : Start value of round trip timer for retry timing

FULLDUP,"@D"    : Fullduplex on modem side

MAXFRAME,"O"    : Maximum number of outstanding packets

MONITOR,"M"     : Frequency monitor functions

MYCALL,"I"      : Callsign of the local operator

With MYCALL the call sign is set permanently on this

channel, with "I" only up to the next disconnect.

A permanent MYCALL can be removed by using "$" as call

sign.

PERSIST,"P"     : Persistance value for transmitter keyup

RESPTIME,"@T2"  : Delay before Info-frame will be confirmed (timer T2)

RETRY,"N"       : Maximum number of retries

SLOTTIME,"W"    : Slottime value for transmitter keyup

TXDELAY,"T"     : Time after transmitter keyup to sending of data

USERS,"Y"       : Number of channel open for connection

XMITOK,"X"      : Locking of transmitter

BUFFERS,"@B"    : Free buffers of TNC

A1SRTT,"@A1"    : A1-value for smoothed round trip timer

A2SRTT,"@A2"    : A2-value for smoothed round trip timer

A3SRTT,"@A3"    : A3-value for smoothed round trip timer

IPOLL,"@I"      : Packet length up to which I-Poll mode is used

VALCALL,"@V"    : Check callsign in connect command

All verbose command can be abbreviated.

1.4.2 External commands

-----------------------

1.4.2.1 Writing data to files

-----------------------------

LOGQSO <filename>	A file will be openend and all transmitted and

received data on the selected channel will be

written.

A CR will be translated to a LF and control-codes

will be translated to ^X. Umlaut-conversion

will be done, too.

The file type ID is 'RN', receive normal.

LOGREC <filename>       Same as LOGQSO but only received data will be

written.

The file type ID is 'RN', receive normal.

LOGSND <filename>       Same as LOGQSO but only transmitted data will be

written.

The file type ID is 'RN', receive normal.

READ <filename>         A file will be openend and all received data will

be written.

Only a CR to LF translation is done.

READ shall be used to receive 7PLUS files.

The file type ID is 'RP', receive plain.

READBIN <filename>      Same as READ but no translation of characters is

done.

The file type ID is 'RB', receive binary.

READABIN <filename>     Start receiving a file using the AutoBIN-protocol.

The file type ID is 'RA', receive AutoBIN.

CLOSE                   Close the active receive-file.

APPEND [ON/OFF]         Flag if data shall be appended to existing files

(only valid for LOGQSO, LOGREC and LOGSND)

1.4.2.2 Sending files

---------------------

SEND <filename>         Send a file on the selected channel, a LF will be

translated to LF.

The file type ID is 'TP', transmit plain.

SENDLOG <filename>      Same as SEND but control-codes will be translated

to ^X and Umlaut-conversion will be done.

The file type ID is 'TN', transmit normal.

SENDBIN <filename>      Same as SEND but no LF to CR translation.

The file type ID is 'TB', transmit binary.

SENDABIN <filename>     Send a file using the AutoBIN-protocol.

The file type ID is 'TN', transmit autoBIN.

BREAK                   Abort sending of a file.

1.4.2.3 Shell and redirect

--------------------------

SHELL [trans]           Open a shell on the current channel.

All received data on the current channel will be

sent to the shell, all data from the shell will

be sent on the current channel.

if no parameter is specified, all linefeed-

characters (0x0A) sent by the shell will be translated

to carriage return (0x0D). All carriage return

characters received from the remote station will be

translated to linefeed (0x0A) prior to sending to

the shell.

The parameter trans disables this conversion, all

characters will be sent transparently.

ENDSHELL                End the shell on the current channel.

REDIR <device>          Redirection of input/output.

All received data on the current channel will be

sent to <device>, all data received from <device>

will be sent on the current channel.

ENDREDIR                End the redirection of the current channel.

1.4.2.4 Directory

-----------------

CD <dir>                Change the working directory to <dir>. If <dir>

is empty, the HOME-directory will be used.

CWD                     Displays the current working directory.

1.4.2.5 Miscellaneous

---------------------

TNC <command>           <command> is sent as a command to the TNC without

any conversion.

CHANNEL <x>             Switch to connect screen of channel x.

or S <X>                If included in a command script, no changing of

screens is performed, it simly specifies the

channel used for the following commands.

XCONNECT <callsign>     Start of a routing script for the specified callsign,

more info can be found in 2.6 .

An active script can be ended by using 'OFF' as

callsign.

SENDCOM <filename>      Execute a file containing TNC or external commands.

(command script)

CBELL [ON/OFF]          Beep if the state of a connection changes.

(connect bell)

INFOBELL [ON/OFF]       Beep if a new packet is received on any channel.

(information bell)

COOKIE [ON/OFF]         Send a cookie if connected from a remote station.

REMOTE [ON/OFF]         Availability of remote commands.

UMLAUT [ON/OFF]         Umlaut conversion. (see 2.2)

CONTEXT [ON/OFF]        Send a special connect text, using macros (see 2.5),

overrides COOKIE.

TXECHO [ON/OFF]         Echo all sent data from input part to part with

received data.

NAME <name>             Set the name of the call on the current channel

to <name>, if <name> is not specified, the stored

name is displayed (only if channel connected).

STIME                   Send time and date on the current channel.

SCOOKIE                 Send a cookie on the current channel.

TIMESET                 Send time of computer to TNC.

DATESET                 Send date of computer to TNC.

RESYNC                  Displays the number of hostmode resyncronisations

since program start.

EXIT                    Leave TNT

2. Special features

-------------------

2.1 Remote commands

-------------------

If remote is enabled (command REMOTE) the remote station can give some commands

to the program. All remote commands start with a double slash '/'. This double

slash must be the first two characters of a received packet, otherwise the

command is not accepted.

The following remote commands are recognized:

//NAME <name>          Store name in database

//SHELL [trans]        Start a UNIX-shell session

//COOKIE               Send a cookie

//DIR <filter>         Show directory

//DIRLONG <filter>     Show directory in long format

//FREE                 Shows space on disks

//INFO                 Info about station

//HELP                 This help-information

//NEWS                 Display news about this station

//READ <file>          Read a file

//BREAK                Abort reading file

//WRITE <file>         Write file to disk

//CLOSE                End writing file

//WPRG <file>          Write file to disk using AUTOBIN-protocol

//WPRG <file> <rfile>  Same as above, but send back //RPRG <rfile>

//RPRG <file>          Read a file using AUTOBIN-protocol

//VERSION              Show version of software

//ECHO <string>        Send back string

//TIME                 Send time

//QUIT                 Disconnect

//DISC                 Disconnect

2.2 Umlaut conversion

---------------------

2.2.1 Display

-------------

If the LINUX-console control codes are used, for a display on the screen

the IBM-characterset is used. No umlaut-conversion will be done.

If termcap is used a received IBM-umlaut is converted depending on UMLAUT.

If UMLAUT is on, the IBM-umlaut is converted into the 8-bit Latin-1 umlaut.

If UMLAUT is off, it is converted to a two character representation ("ae").

2.2.2 File-receive

------------------

Only log-files (LOGREC, LOGSND and LOGQSO) are affected by umlaut conversion.

If UMLAUT is on, the IBM-umlaut is converted into the 8-bit Latin-1 umlaut.

If UMLAUT is off, it is converted to a two character representation ("ae").

2.2.3 File-send

---------------

Only log-files (SENDLOG) are affected by umlaut conversion.

If UMLAUT is on, the 8-Bit Latin-1 umlaut is converted into the IBM-umlaut.

If UMLAUT is off, it is converted to a two character representation ("ae").

2.3 Shell-login and Redirection

-------------------------------

TNT allows a remote user to log into UNIX as a normal user. All received

data is treated as shell input, all data from the shell is transmitted to

the remote station.

The shell can be started by the operator using command SHELL or by the

remote station using command //SHELL.

It is possible to execute all programs which are permitted to use from

the shell.

If you start a shell on a remote system, all programs which need a special

terminal lead to problems in the screen output. Therefore it is possible

to redirect to a virtual console (or any other device, file, pipe, e.t.c.).

The control codes for a terminal are interpreted correctly by the virtual

console and all data entered will be transmitted.

Using this feature screen oriented programs can be executed on the remote

system using TNT as transport medium.

If you want to use this feature be sure to give parameter 'trans' with the

SHELL command. Otherwise there will be a linefeed/carriage return conversion

active which lead to problems using screen oriented programs.

To increase performance, for shell and redirection all data from the shell

or the redirection console will be buffered.

This means that data is not sent directly, but if the buffer contains 256

Bytes (the AX25 maximum packetlength) or if for a specific time no new

characters were received. This time can be configured in the initialisation

file.

2.4 AutoBIN: File transfer

--------------------------

To transfer binary files easily without much overhead but a with a security

check, the AutoBIN-'protocol' was defined.

It is implemented in many PR-programs. Therefore it is implemented in TNT,

too. To use AutoBIN the commands SENDABIN and READABIN are available for

the operator and //WPRG and //RPRG for a remote user.

At the end of a successful transfer the elapsed time and the effective

Baudrate is displayed. If a file was received, the received checksum and the

calculated checksum will be displayed, too.

If the transfer was aborted, the connection was disconnected or the

calculated checksum is not equal to the received checksum, the received file

will be moved to a special directory (abin_dir). In addition the name is

changed to a unique name. From time to time it is needed to clean up this

directory.

Although in most cases these corrupted files are of no interest, for the

rare cases where they are needed they are kept in this directory.

2.5 Special connect text with macros, Name-database

---------------------------------------------------

To be able to send a special connect text to a connecting station, macros

can be included in the connect text file.

The connect text file will be sent on a connect if CONTEXT was set to ON.

For default the connect text file (tnt_ctextfile) in the tnt directory

will be used, but you can put for some users a personal connect text in

the connect text directory (ctext_dir). The file must be named

<callsign>.ctx .

To be more personal and to help remembering the name of the operator of the

remote station a name database is included.

A name can be entered by the operator using the NAME command or with //NAME

given by the remote station.

If no name is specified after the command, the current name will be

reported.

Following macros are expanded, if found in a connect text file:

%v: Version of TNT running

%c: Call of other station

%n: Name of other station (extract from names database)

%y: Call of own station

%k: Channel number

%t: Current time

%d: Current date

%b: Bell (^G)

%i: Include news-file ('news_file_name')

%z: Timezone

%_: CR/LF

%o: Include a cookie

%?: Send a message if name of other station not contained in

names database

%%: '%' itself

2.6 Routing scripts

-------------------

The structure of the Packet Radio Network is quite different compared to

the internet. In the internet you can directly specify your destination and

the routing will be done automatically.

In the Packet Radio Network there exist different systems with incompatible

routing mechanisms. To connect to a destionation there may be needed several

steps. Suppose the following example:

You want to connect DL7ZZZ which is standby on the frequency of the digi-

peater DB0LUC. Your local digipeater is DB0BLO.

Therefore your first command is to connect DB0BLO (:c db0blo). If the link

setup was successful you will send the line 'c db0ber' to the digipeater

which then will build up a connection to DB0BER, which is the next digi-

peater towards DB0LUC. If the connection is established, DB0BLO will send

you the message: '*** connected to DB0BER'.

Now you enter the line 'c db0luc db0bln', which means DB0LUC can be reached

from DB0BER via DB0BLN. DB0BLN is not connected, because it uses hop-to-hop

acknowledge instead of simple digipeating.

Now if the connection was successful, you get the line

'BSUED:DB0BER> Connected to DB0LUC via DB0BLN' and you can at last try to

connect your destination with the line 'c dl7zzz'. The successful connection

is indicated by the line '*** connected to DL7ZZZ'.

If you do this manually, you must look at your screen and after each

successful connect-message you must enter the new line.

The connect-script facility (command XCONNECT) now does all these steps

automatically, if you have given the routing information to the program.

The routing information will be searched in the names-database file

(name_file_name) and reads for the example as follows:

T>DL7ZZZ Joe; N>DB0LUC T>DL7ZZZ

N>DB0LUC Digi Luckau; N>DB0BER F>DB0BLN N>DB0LUC

N>DB0BER Digi Tempelhof; N>DB0BLO N>DB0BER

N>DB0BLO Digi 9K6

As you can see the routing entries are recursive, you don't have to specify

the whole routing for the destination.

The characters before the callsigns give the information to the connect

script shall connect the digipeater ( N> ) or if the digipeater shall be

used in the via path ( F>, D> ).

To identify a normal user T> is used, for mailboxes B> can be used.

Some programs used by normal users allow a connect with the command

//c <callsign>. If you want to use such a user as part of your path,

put a T> in front of the callsign and a '//c <callsign>'-line will be

send by the routing script.

2.7 Call update

---------------

If you use routing scripts or if you connect manually through a large

number of digipeaters, you easily loose the orientation, where you are.

Therefore, every 'connected to' line will update the callsign displayed

in the statusline.

'reconnected to' lines will update the call in the statusline, too, but

routing scripts will not send the next command line.

2.8 Logbook

-----------

All connections performed by the station are written into a logbook-file.

Each line contains starttime and endtime of the connection and the callsign

of the remote station. If the destination was connected directly only this

call is logged, if the destionation was connected using several digipeaters

or by a routing script, the first digipeater used is logged, too.

The name of the logfile can be specified in the init-file (tnt_logbookfile).

Example:

Starttime   |    Endtime     |          Callsign

-------------------------------------------------------------------------------

16.03.94 18:08 | 16.03.94 18:09 | DB0BLO

17.03.94 21:32 | 17.03.94 21:37 | GEHREN:DB0LUC, Uplink: DB0BLO

3. Installation

---------------

3.1 Configuration file

----------------------

The configuration of the program is done by a configuration file.

This file called 'tnt.ini' is searched at the current directory or at the

home directory of the user (other filenames can be specified by using the

-i option at startup).

sample tnt.ini:

# device to which TNC is connected

device /dev/cua0

# lockfile for device

tnt_lockfile /usr/spool/uucp/LCK..cua0

# baudrate used

speed 19200

# username for remote permissions

remote_user guest

# timeout packet assembly for SHELL/REDIR (seconds)

pty_timeout 2

# tnt directory

tnt_dir /work/tnt/

# remote directory

remote_dir /work/tnt/remote/

# ctext directory

ctext_dir /work/tnt/ctext/

# directory for corrupt autobin-files

abin_dir /work/tnt/abin/

# upfile

tnt_upfile tnt.up

# downfile

tnt_downfile tnt.dwn

# infofile

rem_info_file tntrem.inf

# helpfile

rem_help_file tntrem.hlp

# cookiefile

tnt_cookiefile /work/cookie/cookies.doc

# namesfile

name_file_name names.tnt

# newsfile

news_file_name news.tnt

# connect text

tnt_ctextfile ctext.tnt

# logbook file

tnt_logbookfile log.tnt

# number of lines for backscroll

# command-screen

lines_command 40

# monitor-screen

lines_monitor 400

# input area of connect-screens

lines_input 20

# output area of connect-screens

lines_output 100

# use color attributes

color 1

# don't use termcap

termcap 0

# attributes for color

# normal characters

attc_normal 0x07

# characters in bottom statusline

attc_statline 0x10

# characters in monitor-headers

attc_monitor 0x06

# characters in channel statusline

attc_cstatline 0x1F

# control-characters

attc_controlchar 0x0F

# remote-answers

attc_remote 0x06

# attributes for monochrom

# normal characters

attm_normal 0x00

# characters in bottom statusline

attm_statline 0x08

# characters in monitor-headers

attm_monitor 0x10

# characters in channel statusline

attm_cstatline 0x10

# control-characters

attm_controlchar 0x10

# remote-answers

attm_remote 0x10

<EOF>

3.1.1 Serial configuration

--------------------------

'device'        : Device, to which the TNC is connected, must be specified.

'speed'         : Baudrate of the serial port

'tnt_lockfile'  : Filename to lock the port

3.1.2 Security

--------------

'remote_user'   : User-Id for remote access.

If tnt was started by root, prior to every remote file command the

user-id is changed to restrict the file permissions of the remote user.

If tnt was started by a normal user, the user-id is not changed. Therefore

a remote user has the same file permissions as the user himself.

3.1.3 Directories and Files

---------------------------

'tnt_dir'        : Directory which contains tnt-files

'remote_dir'     : Directory used for remote access

'ctext_dir'      : Directory used for personal connect texts

'abin_dir'       : Directory for files of unsuccessful AutoBIN-transfers

'tnt_upfile'     : Command script executed at startup of program

'tnt_downfile'   : Command script executed before exit of program

'rem_info_file'  : File transmitted if remote command //info is received

'rem_help_file'  : File transmitted if remote command //help is received

'tnt_cookiefile' : File which contains database for fortune cookies

'tnt_logbookfile': File for logbook

'name_file_name' : File for names database and routing scripts

'news_file_name' : File for news message, transmitted if //news is received

or if %i macro is found in a connect text file

'tnt_ctextfile'  : File containing common connect text

All files except 'tnt_cookiefile' are searched in the directory specified

by 'tnt_dir'. 'tnt_cookiefile' must contain the complete path to the file.

3.1.4 Lines of virtual screens

------------------------------

'lines_command' : Number of lines of the command screen.

'lines_monitor' : Number of lines of the monitor screen.

'lines_input'   : Number of lines of the input part of the connect screen.

'lines_output'  : Number of lines of the output part of the connect screen.

3.1.5 Display configuration

---------------------------

'color'         : non color attributes (0) or color attributes (1)

'termcap'       : use LINUX-console control codes (0) or /etc/termcap (1)

if color and LINUX-console control codes are selected, but the TERM entry

in the environment is not "console", no color and termcap is used.

3.1.6 Screen attributes

-----------------------

Depending on the selected mode (color) the color or monochrom attributes

are used.

Important notice: The attributes for normal text and for controlcharcters

in the input field MUST be different, otherwise problems with the inter-

pretation of the characters will occur.

3.1.6.1 Color attributes

------------------------

'attc_normal'     : For normal text.

'attc_statline'   : For main statusline at the bottom of the screen.

'attc_monitor'    : For monitor headers.

'attc_cstatline'  : For statusline of channel in connect screen.

'attc_controlchar': For controlcharacters in the input field.

'attc_remote'     : For text transmitted because of remote functions.

The attributes are coded in the following manner:

X    X    X    X    X    X    X    X

Blink BCo2 BCo1 BCo0 FCo3 FCo2 FCo1 FCo0

Background-      Foreground-

Colour           Colour

Blink:

0:off, 1:on

Background Color:

BCo2 BCo1 BCo0

Black     0    0    0

Red       0    0    1

Green     0    1    0

orange    0    1    1

blue      1    0    0

magenta   1    0    1

cyan      1    1    0

white     1    1    1

Foreground Color:

FCo3 FCo2 FCo1 FCo0

Black           0    0    0    0

Red             0    0    0    1

Green           0    0    1    0

Orange          0    0    1    1

Blue            0    1    0    0

Magenta         0    1    0    1

Cyan            0    1    1    0

White           0    1    1    1

Grey            1    0    0    0

Light red       1    0    0    1

Light green     1    0    1    0

yellow          1    0    1    1

Light blue      1    1    0    0

Light magenta   1    1    0    1

Light cyan      1    1    1    0

Light white     1    1    1    1

3.1.6.2 Monochrom attributes

----------------------------

'attm_normal'     : For normal text.

'attm_statline'   : For main statusline at the bottom of the screen.

'attm_monitor'    : For monitor headers.

'attm_cstatline'  : For statusline of channel in connect screen.

'attm_controlchar': For controlcharacters in the input field.

'attm_remote'     : For text transmitted because of remote functions.

The attributes are coded in the following manner:

X    X    X    X    X    X    X    X

Att5 Att4 Att3 Att2 Att1 Att0

Att0: Standout    (termcap so/se)

Att1: Underline   (termcap us/ue)

Att2: Blink       (termcap mb/me)

Att3: Reverse     (termcap mr/me)

Att4: Bold        (termcap md/me)

Att5: Halfbright  (termcap mh/me)

0: off, 1: on

3.1.7 Packet assembly timeout

-----------------------------

'pty_timeout'     : Time for packet assembly

All characters received by a shell or redirection are buffered. If the

buffer reaches the maximum packetsize (256 bytes) the buffer is transmitted.

After each new character pty_timeout seconds are waited for the next

character. If during this time no character is given, the buffer will be

sent even if the maximum packetsize is not reached.

3.2 TNC-configuration files

---------------------------

At the startup of the program a command script is executed. It can be used

to initialize some parameters in the TNC. The filename must be specified

after 'tnt_upfile' in the init file.

To set up different callsigns on specific channels the following command

sequence can be used:

...

CHANNEL 4

MYCALL DL4YBG-4

CHANNEL 5

MYCALL DL4YBG-5

CHANNEL 6

MYCALL DL4YBG-6

...

After the EXIT command is given another command script is executed before

the termination of the program. The filename must be specified after

'tnt_downfile' in the init file.

3.3 Cookie file

---------------

If fortune cookie sending is active (command COOKIE) and someone is connecting

to the station a randomly selected cookie out of the cookie file will be sent.

The cookie file consists of cookie texts delimited by a line with a '-' as

the first character. The minimal size of a cookie file is 2048 bytes.

The filename of the cookie file must be given after 'tnt_cookiefile' in the

init file.

3.4 Files for remote commands

-----------------------------

If the remote command //help is received a file specified after

'rem_help_file' in the init file is transmitted.

The file shall contain an overview and an explanation of the possible remote

commands.

If the remote command //info is received a file specified after

'rem_info_file' in the init file is transmitted.

The file shall contain a description of the station and the used equipment.

If the remote command //news is received a file specified after

'news_file_name' in the init file is transmitted.

The file shall contain any news about the station.

3.5 Files for connect text

--------------------------

If connect text sending is active (command CONTEXT) and someone is connecting

to the station a special connect text will be sent.

The connect text is fetched from the file specified after 'tnt_ctextfile'.

Any macros contained in the file will be expanded.

It is possible to send several users a personal connect text. To achieve this

a file called <callsign>.ctx must be put to the directory specified after

'ctext_dir', where <callsign> is the call of the user.

3.6 Files for names database and routing scripts

------------------------------------------------

For use with the connect text feature a names database is included. The

database itself is contained in the file specified after 'name_file_name'.

The same file is used for the database for the routing scripts.

The entries for routing scripts can be recursive, it is not needed to

specify the whole path for each callsign.

The path is assembled using the relevant entries in the database.

In front of the callsign a letter specifies the type of the station:

T : Normal user

N : Network node

All other characters are accepted.

The only difference is the treatment of the call in routing scripts.

The database is a text file containing lines like the following:

T>DL4YBG Mark

-> The name of the operator of station DL4YBG is Mark, no routing information

given or station can be reached directly.

T>DL7ZZZ Joe; N>DB0LUC T>DL7ZZZ

-> The name of the operator of station DL7ZZZ is Joe, the station can be

connected from digipeater DB0LUC.

N>DB0BLO Digi 9K6

-> The digipeater DB0BLO can be connected directly.

N>DB0BER Digi Tempelhof; N>DB0BLO N>DB0BER

-> The digipeater DB0BER can be connected from digipeater DB0BLO.

N>DB0LUC Digi Luckau; N>DB0BER F>DB0BLN N>DB0LUC

-> The digipeater DB0LUC can be connected from digipeater DB0BER via DB0BLN.

3.7 User-Id's and security

--------------------------

For every user it is possible to log into the UNIX-system by using //SHELL

or if the operator uses the SHELL command for the channel.

The call sign of the connected station is used as the user-id.

If there is no user-id existing with this call sign, the user-id "guest"

is used.

Because at the moment there is no security for the password, no passwords

are used.

Therefore the entries in /etc/passwd shall look like:

guest::407:109::/home/guest:

dl4ybg::405:101::/home/dl4ybg:

These user-id's must not appear in /etc/shadow.

Check the file permissions on your system, now you are not alone any longer!

4. Options at startup

---------------------

Usage : tnt [-i <init-file>] [-l <log-file>] [-u]

TNT has three startup options. A file after -i is taken as the init file and

a file after -l is opened on channel 1 equal to a LOGREC command. If the

program was not normally ended and the serial port is locked, the lockfile

can be ignored with -u.

The -l option is useful if the TNC holds data from the time without

a terminal.

5. Running under X11

--------------------

TNT needs a XTERM console for execution. Therefore TNT can be started from

every XTERM shell.

A sample script file 'xtnt' can be used to create a new window for TNT. To

change the parameters in 'xtnt' see the manual entry for XTERM.

6. Porting of TNT

-----------------

In config.h you can specify, if your system has usleep() available and

if you want to use select() instead of polling with usleep().

The codes generated by function and special keys are defined in keys.h and

must be adapted.

Because in most cases not all keys are listed in the /etc/termcap entry the

keycodes are defined in this way.

If your porting was successful, please send me your changes. They will be

included then in the next release.

7. The end

----------

Thanks to Joerg, DD8FR @ DB0EAM for providing the code for macro expansion

in the connect text.

Thanks to all who have tested TNT and have given comments.

If you have questions, comments or bug reports, just write a mail:

Ham Radio : DL4YBG @ DB0BLO.#BLN.DEU.EU

Internet  : wahlm@zelator.de

73, Mark Wahl (DL4YBG)