SLOW SCAN TV

using the

SOUND BLASTER

Required equipment:

IBM (or compatible) computer, the faster, the better!

Math co-processor not necessary, but also is nice, as the pictures

display faster.

VGA display, capable of 640 x 480 - 256 colors. This requires a VGA

card with at least 512K memory.

Sound Blaster (or compatible) sound card. The program has been

tested with the Sound Blaster V1.5, V2.0, and Pro, as well as Media

Visions Pro Audio Spectrum 16 and Fusion 16 (let me know of others

as you try them!).

This program is designed to display 8 second BLACK & WHITE slow

scan pictures that you can copy off the ham radio bands. The

frequencies that you will normally find slow scan are:

3.845  MHZ

7.171  MHZ

14.230 MHZ

21.340 MHZ

28.680 MHZ

The most popular frequency is on 20 meters at 14.230 MHZ, with

14.233 MHZ being used on weekends when there are too many on

14.230.

The following diagrams might help with the electrical hookup. For

receive, the audio from your receiver needs to be connected to the

microphone input on the Sound Blaster. The audio from your receiver

may come from the speaker output. In my case, I used an audio

output from the back of the radio (Icom 730) that is to be used for

RTTY, Slow Scan, packet, etc. The audio out of the back does not

vary with the volume control, so I can keep the audio at a

reasonable level and still copy pictures.

For transmit, take the audio out (one channel, left or right) to

the microphone on your transmitter. Again, I went into the back of

my radio. With the IC730, you have to use an amplified microphone.

Mine just happens to have a level control under the base so the

level into the microphone can be adjusted down to keep the room

noise from interfering with the picture. Use your microphone button

to key up the transmitter.

Sound Blaster Board V1.5 and V2.0

________________

|

|<---- Sound in from receiver

|

Volume -->|

Control   |

|----> Sound out to your microphone input

|

|

|<---> Midi input/output (not used for this

|      project)

________________|

Sound Blaster Pro

________________

|

|<---- Line in (not used for this project)

|<---- Microphone in - Sound in from receiver

Volume -->|

Control   |

|----> Sound out to your microphone input

|      for transmitting Slow Scan

|

|<---> Midi input/output (not used for this

|      project)

________________|

I did the development on the Sound Blaster Pro, but I kept the

sample rate at 12 KHZ (the top speed for the Sound Blaster V1.5 and

V2.0) so all Sound Blasters could be used. It seems to give

acceptable results at this speed, however, a faster sample rate

would give even better pictures. In a future version, if there is

interest, I may make a version with extra options just for the Pro

version.

Now that the program has been out for a short while, I've received

feedback from others who inform me that it also works on the PAS16

(Pro Audio Vision from Media Vision) and the Fusion 16 (also from

Media Vision). In this version I am packing the drivers, as some of

the cards, including the V1.5 and V2.0 from Creative Labs, do not

come with the ones needed. Make sure to read about the drivers as

they have to be set up just right to make the thing work.

Copy the program to your hard disk (preferably make a separate

subdirectory called SS). You need all the BGI files for the program

to run. Your computer should have several meg of free space as the

.VOC files will take up about 100k for every 8 seconds of

recording. Also you will want to save some of your favorite

pictures in .PCX files so you can recall them later. The reason I

chose the .PCX format is that it generally a lot less space than

.VOC files. The display on your computer needs to be a VGA display

that will work in 640 x 480 - 256 color mode. Some VGA cards do not

have enough memory to do this. If it doesn't work on your system,

I need to know so I can make the necessary changes. When reporting

problems, please make sure to include as much information about

your computer system as possible (autoexec.bat, config.sys, path,

etc), so I can be of help.

The first time that you run the program type "ss8". You will see a menu

of choices for your video card. Select the one that matches the

video card in your computer. If none match exactly, try number 11,

which is a generic driver. Only when you select 11 will you see a second

menu which will choose another driver for the .PCX files only. In

other words, if the program runs OK except for not being able to save

.PCX files, the driver on the second menu is not correct for your video

card. All of the other choices will not have a second menu. The selections

that you make are saved in a file called "SS.CFG". The next time that you

run the program, you will not be given the choices as it knows from the

file "SS.CFG" what you selected the first time. If you need to make

changes, you can:

1. Erase the "SS.CFG" file from your hard drive, or

2. Type "ss8 v" instead of "ss8" to start the program. This will

automatically erase "SS.CFG" so you can make your video

selections again.

Make sure that in your autoexec.bat that you have the following:

SET SOUND=C:\SBPRO

SET BLASTER=A220 I7 D1 T2

The first line above is telling the computer where your Sound

Blaster programs are located. If your directory name is different

that "SBPRO" then you need to insert the correct directory name

(and throughout the next few paragraphs).

In a subdirectory under "SBPRO" should be a directory called "DRV".

C:\SBPRO\DRV

You MUST have a driver called "CTVDSK.DRV" located in this DRV

subdirectory.

C:\SBPRO\DRV\CTVDSK.DRV

If you do not have this driver, I have included it for to copying

into the DRV subdirectory. However, you must copy the right one for

your card. There are three drivers included:

ctvdsk.v15      for the Sound Blaster version 1.5 & SBMCV

ctvdsk.v20      for the Sound Blaster version 2.0

ctvdsk.pro      for the Sound Blaster Pro

If you do not have a file named CTVDSK.DRV in you DRV directory,

you must rename the proper one above  to "CTVDSK.DRV" and copy it

to your DRV directory.

The following is an example of how to copy and rename the file in

one step:

COPY ctvdsk.v15 C:\SBPRO\DRV\ctvdsk.drv <enter>

The second line above sets the BLASTER environment which is read by

the program.

The A220 should be the address that you have set your Sound Blaster

at. Other choices could be A210, A230, A240, A250, and A260. The

Pro only has two choices, A220 and A240.

The I7 is the interrupt of the card. Other choices could be I2, I3,

I5, or I10. I would guess that if you already have the card up and

running with other applications, you only need to find out what

you are using.

D1 is the DMA channel used. I believe that in the Sound Blaster

V1.5 and V2.0, you can only use DMA1. However, on the PRO you can

use DMA 0, 1, or 3. DMA 1 is the default.

T2 is the parameter that informs the program which Sound card is

installed.

The choices are:

1 is for SB V1.5 or SBMCV

2 is for SB PRO

3 is for SB V2.0

All of the above are jumpers that you put in place on your Sound

Blaster at the time you installed it in you computer. If you have

questions about what yours is set at, Creative Labs provided a

program called "TEST-SBC.EXE" that gives you the information you

need ("TEST-SBP.EXE" for the PRO). Your Sound Blaster manual will

give you further details.

To start the program after it is loaded type "SS8" ("SS8 V" to reselect

the video configuration.

Once you have the audio connected, and a station ready to transmit,

pressing F2 will start the receiving process. The sound is stored

in a disk file named "TEMP.VOC". This file is overwritten every

time you press F2. You will not see anything happen until you press

ESC which should be pressed as soon as the audio for the picture

stops. The "TEMP.VOC" file will be a minimum of 100k in size for

one 8 second period, so you will need to make sure that you have at

least 500k free on your hard disk where ss.exe is loaded (several

megabytes would be even better). You will learn after using the

program that it takes a little bit before it starts recording, so

you need to anticipate the start of the picture just a little.

If you anticipate too much, you can do a couple of different

things:

1. Press ESC will stop the recording & start to display whatever

there was to that point

- Pressing ESC again will stop the display

- Pressing F2 will restart the recording.

2. If, after you are displaying the picture, you see that the top

of the picture is part way down on the screen,,pressing the space

bar will make the rest of the picture restart from the top so you

will see a complete frame.

Note: I know that it is pretty slow by not copying the picture to

the screen as it is coming in, but that may be changed in a future

version.

Once you have a picture on the screen, if you Press F2 again you

will loose the one you have. Menu item F7 allows you to save the

picture as a .PCX file, or you can rename the "TEMP.VOC" file to

another name so when you press F2 again, you do not loose the

picture. I have, just for the fun of it, looked at the .VOC file

and edited them in VEDIT which comes with the Sound Blaster.

Even though I wrote the program code for 8 second Black & White, I

have been able to receive 36 second, 72 second, & SCOTTY 1 in Black

& White. A portion of the picture gets thrown away (you only see

the left hand upper corner) but, I was quite surprised when I saw

how well it did.

Transmit has been added with two options.

1. Pressing F4 will transmit "TEMP.VOC", the last picture

received.

2. Pressing F5 will give you a menu of ".VOC" files that

you have saved. By saving the "TEMP.VOC" file to other

names, you can later re-transmit the pictures you have

received. It's just like a tape recorder!

Help is built into the program and accessed by pressing F1. This

will give you a second menu of help items. Just press the F key

that will give you the information that you want.

Test images are built in, but, other than nice to look at, are of

little use until there is a send program (one that reads the

screen) written.

The CONTRAST, SYNC, and BLACK can be adjusted if desired. This is

left over from my development stages & was left in. It may give you

more insight into how a Slow Scan picture works by playing with

these. Once you have recorded and have a "TEMP.VOC" file, you can

use this file under the F7 option to re-display as many times as

you like using the SYNC, CONTRAST, and BLACK. For those not

familiar with Slow Scan, there are many different modes. An

excellent discussion is in the January 1993 issue of QST.

Updates from V1.0 to V1.1

Fixed 'divide by zero' when no audio was coming in during receive.

Incorporated *.CHR files in .EXE program.

Added transmit capability by re-transmitting .VOC files.

Included drivers for Sound Blaster in .ZIP package.

Updates from V1.1 to V1.2

Added a video configuration menu to handle more types of VGA boards.

Updates from V1.2 to V1.3

More video support for Diamond Speedstar and Stealth  (use VESA)

If you can use the VESA driver, that's the way to go.

Updates from V1.3 to V1.4

Divide by zero re-appeared... Should be ok again.

Cleaned up the white colors, some appeared gray.

Updates from V1.4 to V1.5

Change made for SB16ASP file format compatability.

I hope this program will give you some enjoyment. If it does, I

would also hope that you will register the program for $20.00. I

have spent over 2 years writing this (not every day mind you) and

it was an exercise for me in learning to program in "C". Well, I

guess something must have worked! Anyway, if I get enough response,

I will get to work and enhance the program with the other modes.

Also, I am interested in any ideas you may have to improve the

program or ideas for other programs for the Sound Blaster. I bought

their development kit, and might as well put it to work. So please

send your $20.00 and ideas for enhancement and other programs you

would like to see (how about copying RTTY or PACKET RADIO with the

Sound Blaster ???). Also, anyone sending in the registration with

$20.00 will receive credit towards any major upgrade to the next

higher level.

Speaking of UPGRADES!!!

SLOW SCAN II for the Sound Blaster is now available! This program

will copy more modes than the original version. It will copy ROBOT

8, 12, 24, 36 second Black and White, and ROBOT 36 and 72 second

COLOR (in Black and White), and Scotty 1 and Scotty 2 (in Black and

White). And, like SLOW SCAN 8 second version, you can transmit the

.VOC files to send pictures as well! The price of SLOW SCAN II is

only $40.00. If you are a registered user, the $20.00 that you have

already paid will be credited, so it will only cost you $20.00

additional.

SLOW SCAN COLOR will be worked on, with a release date expected

early 1994. Depends on how busy I am at my "real" job!

I will also be working on copying REAL TIME. No promises yet,

though I should be able to do that.

Comments or questions can be left for me on Compuserve. Make sure

that you address to Gene Harlan  76645,2455.

Questions can be answered evenings at (815) 398-2683. I'm usually

home by 6:00 pm (CST) unless I have meetings, etc. I have enjoyed

helping those that have called to date. All the comments and

compliments were appreciated!!! Compuserve is probably easier and

I try to check it once a day when I'm in town.

Please send the following registration form to:

Gene Harlan - WB9MMM

5931 Alma Dr.

Rockford, Illinois 61108

Registration Information

_________________________________________________________________

______ I have enclosed $20.00 and would like to register the Slow

Scan program.

______ I am a registered user and would like to upgrade to Slow

Scan II for $20.00, which copies 8, 12, 24, 36 second Black

and White and 36 and 72 second color (in Black and White)

and Scotty 1 and Scotty 2 (in Black and White). Please

include $5.00 shipping (Illinois residents include $1.25

tax.

______ I am not a registered user yet and would like to get the

Slow Scan II program. I am enclosing $40.00 plus $5.00

shipping (Illinois residents include $2.50 tax).

______ I am not a registered user, and cannot afford it at this

time. However, I like the shareware version and would like

to be on your mailing list. If I use the program, as soon as

I can afford to send money, I will. In the mean time, I'll

get on the air and tell everyone how great it is!

_______NEW!!! Audio Analyzer - View the waveforms that you hear for

Slow Scan, RTTY, Amtor, Packet, ect. See the sync pulses and

be able to measure the time in milliseconds. This program

is only $39.95 plus $5.00 shipping (Illinois residents include

$2.50 tax) Real time samples can be taken or record and playback

from .VOC file.

_______SPECIAL - Both Slow Scan II and Audio Analyzer - ONLY $69.95!

Save $10.00. (Shipping $5.00 - Illinois residents add $4.37 tax.)

This offer subject to change at any time.

VISA & MASTERCARD ACCEPTED on phone orders! Call Today!

Name_________________________________________

Call Sign______________________

Street_________________________________

City___________________________________ State __________

Zip__________   Phone_____________________________

Type/speed of computer __________________________________

Size of hard drive ______________________________________

Type of display card ____________________________________

PLEASE BE SPECIFIC ABOUT YOUR DISPLAY CARD - THIS WILL HELP ME!

Ram - how much do you have ______________________________

Sound Blaster _____ Sound Blaster Pro _____ SB20 _____ SB16______

PRO AUDIO SPECTRUM 16_____ FUSION 16 _____ OTHER __________________

3 1/2 disk _____  5 1/4 disk _____

Other information - ideas - suggestions

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________